Section Cover Page

Section 12 48 13 2011-08-26 Entrance Floor Mats

Refer to “LEED Notes and Credits” page for additional guidance for LEED projects. Delete LEED items if project: .1 is excluded by the Department’s policy on LEED, or .2 the Department has determined that the work of this Contract is not to attain a LEED rating.

This Master Specification Section contains:

.1	This Cover Sheet

.2	LEED Notes and Credits

.3	Specification Section Text:

1.	General

1.1	Related Work Specified in Other Sections

1.2	Reference Documents

1.3	Sequencing and Scheduling

1.4	Submittals

1.5	Waste Management and Dispoal

1.6	Warranty

2.	Products

2.1	Materials

3.	Execution

3.1	Examination

3.2	Installation

3.3	Protection

Schedule out Entry Mat Locations by room name and number, quantity and size variation if any.

LEED Notes:

N/A

LEED Credits:

Contribution towards LEED credits in this section may apply as follows:

.1	LEED Credit MR 4: Recycled Content:

.1	The criteria is that the sum of post-consumer recycled content plus one-half of the pre-consumer content constitutes at least the percent (10% and 20%) of the total value of the materials in the project. Steel and aluminum components have some recycled content and will contribute to the overall recycled content of components in the building. The manufacturers will be able to provide information on the content of recycled content in their product.

2.	LEED Credit MR 5: Regional Materials:

.1	If fabricated regionally, products specified will apply towards this credit. The contractor will be required to submit documentation consisting of cost, weight, transportation service and distance the material has travelled to the construction site.

3.	LEED Credit EQ 5: Indoor Chemical and Pollutant Source Control:

.1	This credit deals with reduction of pollutants from the exterior through employment of permanent entryway systems such as mats, grilles and grates to capture dirt and particulates from entering the building at all high volume entryways. Product Data Sheets will be required for submission in the event of an audit.

1.	General

1.1	RELATED Work Specified in Other SECTIONS

.1	[LEED Submittal Forms	Section 01 32 16]

.2	[LEED Requirements	Section 01 35 18]

.3	[Environmental Procedures	Section 01 35 20]

.4	Waste Management and Disposal	Section 01 74 19.

.5	Cast-In-Place Concrete	Section 03 30 00.

.6	Resilient flooring:	Section 09 65 00.

.7	Carpeting:	Section 09 68 17.

1.2	REFERENCE DOCUMENTS

SPEC NOTE: Edit this article to include only standards referenced within the edited version of this Section, including LEED documentation and sustainable practices.

.1	American Architectural Manufacturers Association(AAMA):

| .1 | AAMA 607.1 | Voluntary Guide Specifications and Inspection Methods for Clear Anodic Finishes for Architectural Aluminum |
| --- | --- | --- |

.2	American Society for Testing and Materials (ASTM):

| .1 | ASTM A276-08a | Standard Specification for Stainless Steel Bars and Shapes |
| --- | --- | --- |
| .2 | ASTM A653/A653M-09 | Specification for Steel Sheet, Zinc Coated (Galvanized) or Zinc Iron Alloy Coated (Galvannealed) by the Hot Dip Process |
| .3 | ASTM A924/A924M-09 | Specification for General Requirements for Steel Sheet, Metallic Coated by the Hot Dip Process |
| .4 | ASTM B 221-08 | Standard Specification for Aluminum and Aluminum-Alloy Extruded Bars, Rods, Wire, Profiles, and Tubes |
| .5 | ASTM D2047-04 | Standard Test Method for Static Coefficient of Friction of Polish-Coated Flooring Surfaces as Measured by the James Machine |

.3	Canada Green Building Council (CaGBC):

| .1 | LEED Canada 2009 Rating System | LEED Canada for New Construction and Major Renovations. LEED Canada for Core and Shell Development. Website: www.cagbc.org] |
| --- | --- | --- |

1.3	Sequencing and scheduling

.1	If product is to be recessed, coordinate with concrete work so that products are available for placing integrally with floor slabs.

1.4	SUBMITTALS

.1	Shop Drawings

.1	Comply with requirements of Division 01.

.2	Shop Drawings:  Show layout and types of grates and frames not less than half-scale sections of typical installations, details of patterns or designs, anchors, and accessories, and field measurements of slab recess to receive frames grates (if applicable).

.2	Product Data:

.1	Comply with requirements of Division 01.

.2	Manufacturer’s product specifications, installation and maintenance instructions.

.3	Samples:

.1	For Selection purposes: submit actual sections of grate and frame material in a convenient but representative size showing full range of colors, textures, finishes and patterns available for each type of floor grate and frame specified.

.2	For Verification purposes: Submit not less than 150 x 150 mm square sections of grate material and 150 mm length of frame material in selected colors and finishes for each type of grate and frame specified.

SPEC NOTE: Delete LEED submittal item if project is not to attain LEED certification or if Credit MR 4 is not being sought.

.4	LEED Submittals: submit LEED submittal forms for Credit MR 4 in accordance with Section 01 32 16 LEED Submittal Forms and Section 01 35 18 LEED Requirements and the following:

.1	Documentation identifying quantity by weight of recycled content in products.

SPEC NOTE: Delete LEED submittal item if project is not to attain LEED certification or if Credit MR 5 is not being sought.

.5	LEED Submittals: submit LEED submittal forms for Credit MR 5 in accordance with Section 01 32 16 LEED Submittal Forms and Section 01 35 18 LEED Requirements and the following:

.1	Regional Materials: provide evidence that project incorporates required percentage [20] [30] % of regional materials/products, showing their cost, distances from extraction to manufacture and manufacture to project site, and total cost of materials for project.

1.5	waste management and disposal

.1	Separate and recycle waste materials in accordance with Section 01 74 19 Waste Management and Disposal and with the Waste Reduction Workplan.

1.6	warranty

.1	Floor mats and frames shall be fabricated free of defects in materials and workmanship in accordance with the General Conditions, and the manufacturer shall offer a 5 year warranty against defects in materials and workmanship.

2.	Products

2.1	MATERIALS

.1	Entry Mat:  no-loop nylon pile permanently bonded to pliable vinyl backing, mat of [    ] mm thick by [    ] mm wide by [     ] mm long sections.

.1	Provide [25] mm wide [black] vinyl border on all edges.

.2	Colour as selected by the Minister [from manufacturer's standard range].

.2	Aluminum Floor Mat

.1	Treadrails from aluminum to ASTM B221, alloy 6063-T5

.2	Inserts: [serrated aluminum][abrasive grit][terrazzo aggregate][carpet of fusion-bonded cut pile].

.3	[Surface mounted with vinyl gradient reducer][Recessed frame].

.3	Aluminum Foot Grilles

.1 	Aluminum Rail Treadbars: to ASTM B221, alloy 6063-T5

.1	Finish: Clear Anodized.

.2	Sound reduction cushions.

.2	Fasteners and rods: Non-corrosive screws, rods and anchors for securing frames together and to floors.

.3	Rail surface dimension: [    ] mm.

.4	Rail spacing: [     ] mm

.5	Tread Insert: [tufted nylon][vinyl][serrated aluminum][abrasive][terrazzo mat]

.6	Recessed Aluminum Frame: extruded aluminum to ASTM B221, alloy 6063-T5 with welded butted corners, anodized finish.

.1	Depth [      ] mm[evaporation pan][drain pan][panless].

.4	Stainless Steel Foot Grilles

.1 	Stainless steel rail treadbars: to ASTM A276, Type 304.

.1	Finish: satin.

.2	Sound reduction cushions.

.2	Fasteners and rods: Non-corrosive screws, rods and anchors for securing frames together and to floors.

.3	Rail surface dimension: [    ] mm.

.4	Rail spacing: [     ] mm

.5	Tread Insert: tufted nylon.

.6	Recessed Frame: stainless steel with welded mitred corners, satin finish.

.1	Depth [      ] mm[evaporation pan][drain pan][panless].

3.	Execution

3.1	EXAMINATION

.1	Substrate must be flat and level to tolerance of no more than 3mm in 3 m.

.2	Examine the substrates and conditions under which the work is to be performed, and notify the Contractor in writing of conditions detrimental to the proper and timely completion of the work. Do not proceed with the installation until unsatisfactory conditions have been corrected.

3.2	INSTALLATION

.1	Entry Mat

.1	Lay mats, after installation of finish flooring and final cleaning and waxing.

.2	Locate mats as follows:

.1	[                 ].

.2	Aluminum Floor Mat

.1	Install products in accordance with manufacturers instructions, at locations shown [and with top of products level with adjoining finished flooring where applicable].

.2	Coordinate top of product surfaces with swinging doors to provide under-door clearance.

.3	Aluminum and Stainless Steel Foot Grilles

.1	Install products in accordance with manufacturers instructions, at locations shown [and with top of products level with adjoining finished flooring where applicable].

.2	Coordinate top of product surfaces with swinging doors to provide under-door clearance.

3.3 	PROTECTIOn

.1	Upon completion of frame installations, provide temporary filler of plywood or fiberboard in grate recesses, and cover frames with plywood protective flooring. Maintain protection until construction traffic has ended and Project is near time of Substantial Completion.

.2	Install product when no further wheeled construction traffic will occur and wet type operations including painting and decorating are complete.

END OF SECTION