Section Cover Page

Section 10165 Plastic Laminate 2000-04-20 Washroom Partitions

Use this section to specify shop-fabricated plastic laminate clad toilet, shower and dressing compartments and screens, which are either overhead braced, floor anchored only, or ceiling hung.

For vandalism prone installations, partitions of more durable materials, e.g. steel and concrete, specifically designed to withstand anticipated abuse, should be detailed on drawings and specified.

This Master Specification Section contains:

1.	This Cover Page

2.	Data Sheets - General

3.	Specification Section Text:

1.	General

1.1	Section Includes

1.2	Related Sections

1.3	Submittals

1.4	Coordination

2.	Products

2.1	Core Materials

2.2	Partition Faces and Edges

2.3	Adhesives

2.4	Hardware

2.5	Bracing, Anchorage and Fasteners

2.6	Fabrication

3.	Execution

3.1	Examination

3.2	Installation, General

3.3	Overhead Braced Compartments

3.4	Ceiling Hung Compartments

3.5	Floor Anchored Compartments

3.6	Screens

3.7	Clean-up

3.8	Schedule

Applicable Standards

1.	ANSI A208.1-1993	Mat-Formed Wood Particleboard

2.	ANSI A208.2-1994	Medium Density Fiberboard for Interior Use

3.	CAN/CGSB-71.19-M88	Adhesive, Contact, Sprayable

4.	CAN/CGSB-71.20-M88	Adhesive, Contact, Brushable

5.	CAN3-A172-M79 (R1996)	High Pressure Paper Base, Decorative Laminates

6.	CSA O112 Series-M1977	CSA Standards for Wood Adhesives

-	CSA O112.4-M1977	Polyvinyl Adhesive for Wood

-	CSA O112.5-M1977	Urea Resin Adhesive for Wood (Room and High Temperature Curing)

-	CSA O112.7-M1977	Resorcinol and Phenol-Resorcinol Resin Adhesive for Wood (Room and Intermediate Temperature Curing)

7.	CSA O153-M1980	Poplar Plywood

8.	CAN3-O188.1-M78	Interior Mat-Formed Wood Particleboard

These reference documents may be available for reference purposes in the Technical Resources and Standards Branch Library, Alberta Infrastructure, 3rd Floor, 6950 - 113 Street, Edmonton; Telephone: (780) 427-7924.

Manufacture

There are several manufacturers of trade name plastic laminate faced partitions with distributors in Alberta.

The work of this Section may also be performed by millwork shops, under the ACTS trade definition for Architectural Woodwork - Supply and Installation.

Partition Types

Floor anchored, overhead braced are recommended on structural concrete floors less than 75 mm thick and on wood floors.  This type has become the most common.

Ceiling hung partitions are typically specified where maintenance, hygiene and operating costs are paramount.  Structural steel fabrications are required to adequately anchor this type of partition.

Floor anchored partitions typically require anchors penetrating minimum 50 mm into concrete floors minimum 75 mm thick; this type is not recommended for anchoring to wood floors.

Floor and ceiling anchored compartments are omitted from this Section but if required they may be specified using a combination of requirements for overhead braced and ceiling hung types.

Core Materials

Plastic laminate washroom partitions with particle board, medium density fibreboard (MDF), plywood, or solid phenolic cores may be specified where greater resistance to vandalism is required than can be obtained with typical metal partitions.

Particle board and MDF are roughly equivalent materials.  Plywood cores offer improved structural properties and moisture resistance compared to particle board and MDF cores, but size limitations require joints in the larger panels.  Plywood also increases risk of warpage, particularly of doors.

Solid phenolic cores offer the best protection against vandalism, are dimensionally most stable, are suitable for wet and humid applications such as shower compartments, and are considerably more expensive.

Finishes

Special mar-resistant coatings over plastic laminates are available on a proprietary basis.  They typically consist of very fine particles of aluminum oxide, a very hard material, distributed in the top phenolic layer in such a manner that colour and texture are not visibly affected.  One manufacturer of this type of coating (Nevamar) states that it does not perform better than typical plastic laminate surfaces in resisting purposeful vandalism such as knife cuts and scrapes.

Pilasters

Pilasters at doors to barrier-free compartments, may require additional stiffening to eliminate vibration from door impact.  One manufacturer suggests attachment of a perpendicular spline matching pilaster construction to the compartment side face of such pilasters.  Another shows an additional pilaster perpendicular to the closed door position.  Designers are advised to consider the options allowed in this Section and to decide whether to provide drawing details in order to control appearance.

Hardware

Hardware for plastic laminate partitions are typically higher quality than hardware typically supplied with metal partitions, presumably due to increased weight of components and greater quality expected.  Hardware is typically unique to each partition manufacturer.  The specification allows for generic hardware and accessories.  Hardware must accommodate pilaster orientation indicated.

Typical Washroom Partition Sizes

Panels and doors are typically made in standard heights of about 1450 mm, located 305 mm above floors.  Overhead braced pilasters are typically 2160 mm high.  Height of partitions by Partition Technics is nominally 2000 mm and gap to floors is typically 160 mm.  Consult with manufacturers before deviating from standard dimensions.

Doors are typically 610 mm wide, 813 mm doors being typically specified for wheelchair accessibility.  Pilasters are typically 300 mm wide.  When a greater degree of privacy is desired, deeper screens and doors can be specified to reduce, but not eliminate, the standard 305 mm gap to floors.  Coordinate with drawings.

END OF DATA SHEETS

1.	General

1.1	SECTION INCLUDES

SPEC NOTE:  Delete this article if toilet compartments only are required.  Edit to suit project.

.1	This Section includes requirements for the following:

.1	Toilet compartments.

.2	Shower compartments.

.3	Dressing compartments.

.4	Urinal screens.

.5	Privacy screens.

.6	[             ].

1.2	RELATED SECTIONS

.1	Steel supports above ceiling for typical ceiling anchorage:	Section 05501.

.2	Fastener backing in wall construction:	Section [     ].

.3	Metal Washroom Partitions:	Section 10162.

.4	Washroom accessories and grab bars:	Section 10811.

1.3	SUBMITTALS

.1	Comply with requirements of Division 1.

.2	Submit shop drawings, clearly indicating fabrication details, plans, elevations, edge details, hardware, and installation details.

.3	Submit large scale details of all anchorages, clearly indicating components, materials, and finishes, and related work.

.4	Submit duplicate 200 mm x 200 mm samples of partition types showing finish on both sides, two finished edges, corners, and core construction.

.5	Submit product data indicating compliance with requirements of this Section.

.6	Submit descriptive literature for hardware items, including, but not limited to, hinges, latch sets, coat hooks, brackets, fastenings and trim.

.7	Provide data describing maintenance of finishes recommended by finish manufacturers for incorporation into Maintenance and Operation Manual.

1.4	COORDINATION

SPEC NOTE:  Delete this article if ceiling anchorage not specified.

.1	Coordinate location of steel supports for ceiling anchorage with work of Section 05501.

.2	Provide templates for locating threaded studs through finished ceilings.

2.	Products

2.1	CORE MATERIALs

SPEC NOTE:  Unless project requirements dictate otherwise (refer to Data Sheets - General), do not restrict choice of core materials, except that solid phenolic may be deleted if not required.

.1	Core Materials:  [except as otherwise specified in Schedule at end of this Section,] provide one of the following:

.1	Particleboard:  to ANSI A208.1-1993, grade 1-M-2 or 1-M-3, or to CAN3-O188.1-M78, Grade R, minimum thickness 25 mm.

.2	MDF:  medium density fibreboard, to ANSI A208.2-1994, minimum thickness 25 mm.

.3	Plywood:  poplar plywood to CSA O153-M1980, standard construction, sanded, minimum grade B veneer face and back, minimum thickness 19 mm.

.4	[Solid Phenolic:  plastic laminate to CAN3-A172-M79, self-supporting grade, colour black, and minimum thickness as follows:

.1	Doors and Pilasters:  19 mm.

.2	Panels:  13 mm.]

2.2	PARTITION FACES AND EDGES

.1	Faces:  high pressure decorative laminate, to CAN3-A172-M79, general purpose grade, standard duty type, 1.14-1.27 mm thick, and as follows:

.1	Finish/Sheen:  velour/suede/matte finish.

.2	Colour [and pattern]:  [as scheduled] [[  ] colours to be selected by Minister from manufacturer's standard range].

SPEC NOTE:  If more than one colour/pattern required, specify locations in Schedule at end of Section.  Specify whether solid colour or printed pattern required since cost varies considerably.

.2	Edges:  one of the following:

.1	Plastic Laminate or PVC Iron-On Type:  [to match colour of faces] [of solid colour contrasting with faces, as selected by Minister from manufacturer's standard range].

.2	Stainless Steel:  partition manufacturer's standard, and as follows:

.1	Type:  wrap-around, sized to be flush with face finishes.

.2	Wall Thickness:  minimum 1.0 mm.

.3	Finish:  type 304, #4 or #6 finish.

.3	Extruded Aluminum:  partition manufacturer's standard, and as follows:

.1	Type:  wrap-around, sized for snug fit against face finishes.

.2	Wall Thickness:  1.5 mm.

.3	Finish:  clear anodized coating to AAM12C22A31, 10-18 micrometre thick.

.4	Exposed Black Phenolic Edge:  polished finish.

2.3	ADHESIVES

.1	Adhesives:  one of the following:

.1	Polymerized urea formaldehyde resin adhesive to CSA 0112.5-M1977.

.2	Resorcinol adhesive to CSA 0112.7-M1977.

SPEC NOTE:  For projects where partitions will be exposed to moisture or high humidity, delete following options.

.3	Polyvinyl acetate resin adhesive to CSA 0112.4-M1977.

.4	Contact adhesive to CAN/CGSB 71.19-M88, sprayable, or CAN/CGSB 71.20-M88, brushable.

2.4	HARDWARE

.1	Hardware not specifically designed for plastic laminate partitions by a partition manufacturer shall be subject to approval by Minister.

.2	Material:  one of the following:

.1	Chrome-plated non-ferrous, die-cast zinc alloy.

.2	Clear anodized aluminum castings and extrusions.

.3	Stainless steel, polished or brushed finish.

.3	Hardware shall operate smoothly, quietly and consistently.

.4	Latch Set, Door Bumper, Brackets:  manufacturer's standard for type of partitions.

.5	Hinges shall be adjustable to automatically return inswinging doors from any position to nominal 30 degrees from closed position, and shall return outswinging doors to closed position.

.6	Hardware for compartments designed for barrier-free access shall be suitable for pilaster orientation indicated on drawings and shall conform to barrier-free design requirements of Alberta Building Code.

.7	Each type of hardware item shall be consistent as to type and finish.

2.5	BRACING, ANCHORAGE AND FASTENERS

SPEC NOTE:  Edit this article to suit project requirements.  Specify stainless steel anchorage and fasteners for shower compartments.

.1	Overhead Bracing:  extruded aluminum with [clear anodized] [    ] finish, stainless steel with No. 4 or No. 6 finish [, or galvanized steel with acrylic finish to match pilasters and doors], of grip-deterring design.

SPEC NOTE:  Specify grip-deterring design for overhead bracing only when located in a space primarily intended for public use.

.2	Floor Anchorage, Including Fasteners:

.1	Concealed:  [steel, hot dip galvanized] [stainless steel].

.2	Exposed:  stainless steel, No. 4 or No. 6 finish.

.3	Ceiling Anchorage:  [steel, hot dip galvanized] [stainless steel].

.4	Panel [and Screen] Anchorage:  continuous extruded aluminum or stainless steel channel to match edging, or extruded aluminum, stainless steel, or chrome plated die-cast zinc alloy brackets.

.5	Floor and Ceiling Anchorage Concealment:  stainless steel sheet, type 302, polished or brushed finish, formed to manufacturer's standard profile.

.6	Exposed Fasteners:  [cadmium plated steel] [stainless steel], tamper-resistant type.

.7	Concealed Fasteners:  [steel, hot-dip galvanized] [stainless steel].

2.6	FABRICATION

.1	Fabricate panels and pilasters to manufacturer's standard heights for type of anchorage and to plan dimensions indicated on drawings.

SPEC NOTE:  Ensure that drawings indicate heights which include all partition manufacturers.

SPEC NOTE:  Include following clause if plywood is a core option specified for this project.

.2	Joints shall only be permitted in plywood cores, as follows:

.1	One horizontal joint permitted in panels supported at both ends.  Locate joints in centre third of panel height, in same panel location for installed panels in same room.

.2	Use corrugated metal joint fasteners and glue joints full length.

.3	Standard compartment doors shall be 610 mm wide; doors to barrier-free compartments shall be minimum 800 wide.

SPEC NOTE:  Ensure that standard door widths are specified above or indicated on drawings, but not both.

.4	Follow recommendations of Appendix A, CAN3-A172-M79, for conditioning materials and for cut-outs in plastic laminate.

.5	Adhere plastic laminate to core material, ensuring support and bond over entire surface and strictly following recommendations of adhesive manufacturer.

.6	Where applicable, apply plastic laminate edging prior to application of facing material.

.7	Provide uniform hairline exposed joints.  Mitre edging corners.

.8	Bevel exposed laminate edges uniformly at approximately 20 degrees.

3.	Execution

3.1	EXAMINATION

.1	Examine site conditions where work is to performed and ensure acceptability of existing room finishes for a complete and satisfactory installation.

3.2	INSTALLATION, GENERAL

.1	Install partitions secure, plumb and square.

.2	Leave 12 mm space between walls and panels, and walls and pilasters.

.3	Fasten anchorage as follows:

.1	To masonry and concrete surfaces using screws and expansion anchors.

.2	To hollow walls using bolts and toggle type anchors.

.3	[To steel supports with threaded rods in threaded holes].

.4	Attach panels and pilasters to brackets with through-type sleeved bolt and nut.

.4	Anchor panels [and screens] to walls and pilasters using continuous channels or consistently spaced brackets.  Provide a minimum of two brackets at ends of each plywood section where panel cores consist of two plywood sections.

.5	Provide for concealed adjustment of [floor] [ceiling] variations.  Conceal floor and ceiling anchorage with stainless steel covers secured in position.

.6	Equip each door with hinges, latch set, door bumper and coat hook.  Adjust and align hardware for easy, proper function.

.7	Set inswinging doors to open position at 30° from closed.  Set outswinging doors to return to closed position.

.8	Install door pulls on out-swinging doors in conformance with barrier-free design requirements of Alberta Building Code.

3.3	OVERHEAD BRACED COMPARTMENTS

SPEC NOTE:  Specify this type of anchorage for typical installations.

.1	Attach pilasters to floor with pilaster supports, level and plumb.  Tighten installation with levelling devices.

.2	Secure headrail to pilaster faces on compartment side, with not less than two fasteners per face.

.3	Set tops of doors parallel with overhead brace when doors are in closed position.

3.4	CEILING HUNG COMPARTMENTS

SPEC NOTE:  Ensure that supporting steel, hangers, threaded rods through ceiling, etc., are specified in Section 05501 and shown on drawings.  Specify plaster key to be installed at plaster ceilings in Section 09206.

.1	Secure pilasters to supporting structural framing using pilaster hangers.

.2	Level, plumb and tighten installation with levelling devices.  Set bottoms of doors level with bottom of pilasters when doors are in closed position.

.3	Assure pilaster hangers do not transmit load to finished ceiling.

3.5	FLOOR ANCHORED COMPARTMENTS

SPEC NOTE:  Specify only where other types of anchorage are unacceptable and anchorage to structural concrete floors is possible.

.1	Secure pilasters to floor with pilaster supports anchored with minimum 50 mm penetration into structural concrete floor.

.2	Level, plumb and tighten installation with levelling devices.

.3	Set tops of doors level with tops of pilasters when doors are in closed position.

3.6	SCREENS

SPEC NOTE:  Delete this article if not applicable.  Edit to suit project requirements.

.1	Provide screens at urinal stalls as indicated on drawings.

.2	Provide screens at [washroom] [      ] entrances, consisting of panels and [floor to ceiling pilasters] [, pilasters and headrails], as indicated on drawings.

.3	Anchor screens to wall [and to [posts consisting of tubular headrail stock and end sockets] [pilasters]], with minimum [2] [3] panel brackets or continuous channel bracket per post or pilaster.  Anchor posts and pilasters to floor [and ceiling].

SPEC NOTE:  Screens at urinals are relatively narrow and, where vandalism is not anticipated, are typically secured to walls only.

.4	Provide supplementary anchorage above ceiling finish to receive [posts] [ pilasters].

3.7	CLEAN-UP

.1	Remove protective maskings and clean surfaces, leaving them free of oil and imperfections.

.2	Field touch-up of scratches or defaced finish will be permitted only if approved by the Minister.  Otherwise defective materials shall be rejected and replaced with new materials.

3.8	SCHEDULE

SPEC NOTE:  Schedule may be deleted if project (1) requires only one type of anchorage, (2) does not require restriction of type of core material and (3) requires only one colour/pattern.  Edit schedule to suit project requirements.

| Location | Function | Anchorage Type | Core Material | Colour/ Pattern |
| --- | --- | --- | --- | --- |
| Rm. [ ] | Toilets | [Overhead braced] | [Particleboard] | [ ] |
|  |  | [Ceiling hung] | [MDF] |  |
|  |  | [Floor anchored] | [Plywood] |  |
|  |  |  | [Solid phenolic] |  |
| Rm. [ ] | Showers | [Overhead braced] | Solid phenolic | [ ] |
|  |  | [Ceiling hung] |  |  |
| Rm. [ ] | Dressing | [Overhead braced] | [Particleboard] | [ ] |
|  |  | [Ceiling hung] | [MDF] |  |
|  |  | [Floor anchored] | [Plywood] |  |
|  |  |  | [Solid phenolic] |  |
| Rm. [ ] | Urinal screens | [As specified] | [Particleboard] | [ ] |
|  |  |  | [MDF] |  |
|  |  |  | [Plywood] |  |
|  |  |  | [Solid phenolic] |  |
| Rm. [ ] | Privacy screens | [As specified] | [Particleboard] | [ ] |
|  |  |  | [MDF] |  |
|  |  |  | [Plywood] |  |
|  |  |  | [Solid phenolic] |  |
| Rm. [ ] | [ ] | [ ] | [ ] | [ ] |
|  |  |  | [ ] |  |
|  |  |  | [ ] |  |
|  |  |  | [ ] |  |

NOTE:	Where more than one core material is specified for a single location and function, provide any one of the materials specified for that location and function.

# END OF SECTION