Section Cover Page

Section 10675 2000-04-20 Industrial Type Metal Shelving

Use this Section to specify industrial type metal shelving and pallet-racking.

This Master Specification Section contains:

.1	This Cover Sheet

.2	Specification Section Text:

1.	General

1.1	Related Sections

1.2	Product Options and Substitutions

1.3	Reference Documents

1.4	Description of System

1.5	Design Criteria

1.6	Performance Requirements

1.7	Submittals

1.8	Delivery, Storage and Handling

2.	Products

2.1	Product Manufacturers

2.2	Materials

2.3	Finishes

3.	Execution

3.1	Assembly of Components

3.2	Installation

3.3	Touch-up and Clean-up

3.4	Installation Schedule

Applicable Standards

.1	ANSI MH28.1-1982	Specification for the Design, Testing, Utilization, and Application of Industrial Grade Steel Shelving

.2	ASTM A653M-96	Steel Sheet, Zinc-Coated (Galvanized) or Zinc-Iron Alloy-Coated (Galvannealed) by the Hot Dip Process

.3	CGSB 1-GP-12c	Paint Colours (Withdrawn)

.4	CAN/CGSB-1.300-M90	Applied Coating System, Semigloss, Baked, for Metal Office Furniture

.5	CAN/CSA-G40.21-M92	Structural Quality Steels

These reference documents may be available for reference purposes in the Technical Resources and Standards Branch Library, Alberta Infrastructure, 3rd Floor, 6950 - 113 Street, Edmonton; Telephone: (780) 427-7924.

1.	General

1.1	RELATED SECTIONS

.1	Custom Steel Fabrications:	Section 05501.

SPEC NOTE:  Include above only when storage shelving will be secured directly to building structure using custom steel fabrications.

1.2	PRODUCT OPTIONS AND SUBSTITUTIONS

SPEC NOTE:  Delete this article if Product Manufacturers article is omitted and non-proprietary method of specifying is used for project specification.

.1	Refer to Division 1 for requirements pertaining to Product Options and Substitutions.

1.3	REFERENCE DOCUMENTS

.1	Shelving shall meet or exceed requirements of ANSI MH28.1-1982, American National Standard for the Design, Testing, Utilization and Application of Industrial Grade Steel Shelving.

1.4	DESCRIPTION OF SYSTEM

SPEC NOTE:  Edit this article to suit project requirements.  Do not include items that will not be required.

.1	General:  Open style, prefinished, self-supporting metal [storage shelving] [and] [pallet-racking] consisting of the following constituent parts:

.1	Shelving:

.1	Uprights.

.2	Shelves.

.3	Kickplates.

.4	Gusset plates.

.5	Braces.

.6	Connectors.

.7	Other necessary components and accessories.

.2	Pallet-Racking:

.1	Uprights.

.2	Load beams.

.3	Shelf panels.

.4	Skid channels.

.5	Drum cradles.

.6	Bracing.

.7	Wall/row connectors.

.8	Corner protectors.

.9	Other necessary components and accessories.

1.5	DESIGN CRITERIA

.1	Method of Assembly:

SPEC NOTE:  Specify desired method of assembly or allow manufacturer option of choosing method.

.1	Shelving:  [boltless [clip] [channel] connectors] [nuts, bolts and washers] [manufacturer's standard method].

.2	Pallet-Racking:  [nuts, bolts and washers] [manufacturer's standard method].

.3	All units shall be easily assembled and completely demountable and relocatable with total salvageability of all components.

.2	Adjustability:

.1	Shelving:  design components to allow for vertical adjustment in 50 mm increments.

.2	Pallet-Racking:  design components to allow for vertical adjustment in 75 mm increments.

.3	Sizes and Configuration:

.1	Shelving Unit - Type 1:

.1	Depth of shelves: [     ] mm nominal.

.2	Length of shelves: [     ] mm nominal.

.3	Height of shelving units: [       ] mm nominal.

.4	No. of shelves per unit: [5] [  ].

.2	Shelving Unit - Type 2:

.1	[           ]

.2	[           ]

.3	[           ]

.4	[           ]

.3	Pallet-Racking Unit - Type 3:

.1	Depth: [         ] mm nominal.

.2	Length: [         ] mm nominal.

.3	Height: [         ] mm nominal.

.4	Levels per unit: [          ].

1.6	PERFORMANCE REQUIREMENTS

.1	Design metal [shelving] [and] [pallet-racking] to support following uniform loads with maximum deflection of 1/180th of span:

.1	Shelving - Type 1: [225] kg per shelf.

.2	Shelving - Type 2: [   ] kg per shelf.

.3	Pallet-Racking - Type 3: [    ] kg per pair of load beams.

.4	[                   ].

SPEC NOTE:  If necessary, consult with manufacturers and specify loading requirements based on their recommendations for specific project.

1.7	SUBMITTALS

.1	Comply with requirements of Division 1.

SPEC NOTE:  Delete above clause if general requirements for submittals are not included in Division 1.

.2	Shop Drawings and Product Data:  indicate shelving layout, components, assembly details, types and thicknesses of materials, finishes and maximum allowable design loads.

1.8	DELIVERY, STORAGE AND HANDLING

.1	Protect finished surfaces from damage during delivery, storage and handling.

.2	Deliver components to site in original unopened containers and packaging bearing manufacturer's seals and labels.

2.	Products

2.1	PRODUCT MANUFACTURERS

SPEC NOTE:  Product manufacturers names should only be included under exceptional circumstances (e.g. new shelving must match existing shelving exactly), and when used, all acceptable manufacturers should be specified.

.1	Metal [shelving] [pallet-racking] by one of the following manufacturers:

.1	[                ]

.2	[                ]

.3	[                ]

2.2	MATERIALS

.1	Sheet Steel:  commercial grade, stretcher levelled to ASTM A653M-96.

.2	Steel Sections and Plates:  to CAN/CSA-G40.21-M92.

2.3	FINISHES

.1	Prepare and finish steel surfaces to CAN/CGSB-1.300-M90.

.2	Paint colour:  [manufacturer's standard colour closest to CGSB 1-GP-12c, colour 501-214, off-white - grey] [match existing shelving colour], semi-gloss.

SPEC NOTE:  Specify to match existing shelving colour if new shelving will be within view of existing shelving, and existing shelving is a standard colour.

3.	Execution

3.1	ASSEMBLY OF COMPONENTS

.1	Unpackage components and assemble units in accordance with manufacturer's instructions.

.2	Assemble in installation location.

3.2	INSTALLATION

SPEC NOTE:  Verify that drawings sufficiently indicate details referenced.

.1	Install assembled units in areas indicated on drawings and in schedule.

.2	Brace, secure and anchor units in place as indicated on drawings.

.3	Secure pallet racking to building structure as detailed on drawings.

3.3	TOUCH-UP AND CLEAN-UP

.1	Touch-up damaged paint surfaces with matching paint, on completion of installation.

.2	Promptly remove debris, surplus materials and tools as work progresses.

.3	Repair damage caused by work of this Section to existing structures and finishes.

3.4	INSTALLATION SCHEDULE

SPEC NOTE:  Include schedule if type, configuration and location of shelving is not indicated on drawings.

| Location | Type | No. of Ranges | No. of Bays |
| --- | --- | --- | --- |
| [ ] | [Shelving] |  |  |
|  | [Single faced] | [ ] | [ ] |
|  | [Double faced] | [ ] | [ ] |
| [ ] | [Pallet racking] |  |  |
|  | [Single faced] | [ ] | [ ] |
|  | [Double faced] | [ ] | [ ] |

END OF SECTION