Section Cover Page

Section 12492 2000-04-20 Window Drapery and Accessories

This Master Specification Section contains:

.1	This Cover Sheet

.2	Specification Section Text:

1.	General

1.1	Quality Guidelines

1.2	Protection

1.3	Site Examination

1.4	Delivery

2.	Products

2.1	Materials

2.2	Drapery Track or Rodding

2.3	Lined Drapery

2.4	Unlined Drapery

2.5	Drapery - General

2.6	Fabric Panels

3.	Execution

3.1	Drape and Track Installation

Applicable Standards

.1	CGSB 4-GP-152Ma	Drapes

.2	CAN/CGSB-86.1-M91	Care Labelling of Textiles

These reference documents may be available for reference purposes in the Technical Resources and Standards Division Branch, Alberta Infrastructure, 3rd Floor, 6950 - 113 Street, Edmonton;   Telephone:  (780) 427-7924.

1.	General

1.1	QUALITY GUIDELINES

.1	The dimensional information on the attached Drapery Requirements Schedule is given to assist the bidder in assessing the scope of the work.  Dimensions must not be used for fabrication.

.2	Printed or patterned fabrics shall have repeats completely matched.

.3	All drapery fabrics and linings shall be inherently flame and burn resistant material in accordance with the Alberta Fire Marshall's regulations.

.4	CGSB 4-GP-152Ma - Drapes.

1.2	PROTECTION

.1	Protect from damage drapery materials and accessories in all phases of handling, fabrication and installation.

.2	Protect from damage all building components during installation.

.3	In the event of damage, repair and make good of no additional expense to the contract.

1.3	SITE EXAMINATION

.1	The sizes indicated in the Drapery Requirement Schedule are based on information available when the Schedule was prepared.  Sizes indicate approximate measurement of finished drape.

.2	Accurate measurements must be made at the site before fabrication.  The building's structural characteristics are to be noted and allowances made for any additional supports, drapery material and track required.  Allowances must also be made for final on-site inspection and adjustments.  All sizes are approximate and to be used for estimating purposes only.   The Contractor shall be responsible in all cases for measurements.

1.4	DELIVERY

.1	Deliver to site at time of drapery installation, tagged as per item schedule.

2.	Products

2.1	MATERIALS

.1	Supply and install all fabric, traverse sets, tracks, hooks, fitments, hardware, weights, etc., which are necessary to complete the installation, unless specified to use existing hardware.

.2	Lining:  [Pride of Paris Roc-lon flame ban], [ivory], unless otherwise specified.

.3	Drapery Fabric, content, colour and pattern as follows:

SPEC NOTE:  Describe drapery fabric selection in preceding.

2.2	DRAPERY TRACK OR RODDING

.1	Standard (1):  Where indicated as Standard on the Drapery Requirements Schedule, the track shall be a "cut to measure, heavy duty centre-close, two-way draw" enclosed track.  Finish white.  Ceiling or wall mount track.  [Kirsch No. 9813], [Eastern No. 2701] rodding sets.  Tracks shall be complete with all necessary accessories including adequate slides, overlap corners, draw cords, wall projection brackets, pulleys, etc.  Tension pulleys to be used, [Kirsch No. 9924].  Finish white.  Tracks used must be of heavy duty construction.

.2	I-BEAM (2):  Where indicated as I-Beam on the Drapery Requirements Schedule, the track shall be [Lilo No. 10] heavy duty steel, "enamel finish", cut to measure track, [Eastern "Star" I-Beam rail].  All tracks shall be complete with all necessary accessories, include adequate rollers, end stops, etc.

.3	Special (A):  Where indicated as Special A on the Drapery Schedule, the track shall be [Kirsch No. 9046] hand traverse, ceiling mounted Architrac, extruded, 1.27 mm, high strength, extruded aluminum alloy single channel track, [Eastern No. 2707].

.4	Special (B):  Where indicated as Special B on Drapery Schedule, the track shall be [Kirsch Paneltrac] complete with precut sliders and one side of ["Velcro"] fastening tape attached, precut aluminum weight strips for bottom hem insertion, pile side of [Velcro] for panel headings and including all necessary accessories including end stops etc.

.5	Special (C):  Where indicated as C on Drapery Schedule, use a Roman blind.  A finished hardwood leader, approximately 25 x 38 mm shaped to accept a [Velcro] tape heading to top surface and screw eyes to underside of Header to be screwed to wall or in case of a recessed window, within the window recess as specified.  Pile side of [Velcro] tape to be applied to blind heading.  Valance, if any to be specified in schedule.  Blinds to be lined, unlined as per Drapery Schedule.  Side seams to be blind stitched.

A flat 10 mm aluminum or brass strip to be sewn into concealed pocket 100 mm above bottom edge of blind.  Bottom 125 mm of blind to be lined with face fabric bottom edge finished straight unless specified as shaped in Drapery Schedule.

Plastic rings spaced at 150 mm vertically and 380 mm horizontally to be securely sewn to back of blind.  Nylon cord knotted to bottom rings and run continuous to tie-off cord fitted with a [#25 Lilo] cord weight with rubber bumper to be used.  Drawing cord to pull over wall-mounted pulley and be secured at wall-mounted awning cleat.

2.3	LINED DRAPERY

.1	Lining to be sewn to face fabric on top and both sides of each drape.

.2	Face fabric return not less than 25 mm.

.3	Lining to be hemmed 25 mm shorter than face fabric.  Hem shall be free hanging.

.4	Drapes shall not be less than double fullness or with widths as specified.

2.4	UNLINED DRAPERY

.1	Fabrics without selvages to have zigzag stitched flat seams.

.2	Selvages of casements to be trimmed, serged and flat sewn along seams.

.3	Hems and headings to be double and not less than 100 mm.

.4	Drapes shall not be less than two and one-half (2 1/2) fullness.

2.5	DRAPERY-GENERAL

.1	Drapes shall be weighted at corners and side seams or shall be continuously weighted with a chain, [as necessary].

.2	Hems shall be machine blind stitched in matching thread of a suitable weight and fibre and flat sewn for best appearance on either side of the drape.  All ends shall be tied off and hems shall be perfectly straight.  Invisible thread not to be used.

.3	Headings shall be stiffened with woven buchram or washable pellon.  Headings to have 3 fold pinched pleats with each pleat tacked top and bottom.

.4	For standard traverse French pleat draperies use "pin-on" type hooks and "sewn-on" end bottom.

.5	Half and full width panels only may be used.  Half width panels shall occur on the outside of each drape and not at the centre.

.6	All drapery shall be centre drawn unless otherwise specified.

.7	Each drape shall be supplied with a permanent sew-in label(s) indicating fabric content and the appropriate CAN/CGSB-86.1-M91 Care Labelling of Textiles.

.8	Refer to drapery track or rodding for specific specifications.

.9	Contractor to consult the Minister before fabrication.

2.6	FABRIC PANELS

.1	Material to be uniformly stretched over 12 x 44 mm wood frame to meet overall dimensions and stapled evenly and firmly on backside of frame so staples are not visible to finished surfaces.  Frame to be reinforced with angle brackets at each inside corner.  Hooks to be centred on back side of top of frame.

.2	All fabrics to be centred horizontally and vertically with full pattern repeats centred whenever possible.

3.	Execution

3.1	DRAPE AND TRACK INSTALLATION

.1	Work shall be executed only by skilled tradesmen.

.2	Prior to installation, draperies shall be pressed to eliminate creases and wrinkles.  Drapes shall be gathered into loose pleats, tied with tape at top, centre and bottom and hung for a minimum of 48 hours.

.3	Install in accordance with manufacturer's specifications, i.e. the correct number of screws, projection brackets, supports, etc.  There shall be no unsupported length over 900 mm.  Where filing or hand drawn, a centre support will be required if track is more than 600 mm long.

.4	All tracks shall be installed using the necessary screws, plugs, toggle bolts, anchors or rawlnuts necessary to produce a heavy duty installation permitting no loosening of brackets, tracks or other operating parts when subject to extensive use.

.5	Drapes shall be hung from [ceilings], [soffits], [or] [walls] as specified.

.6	Hems shall be perfectly straight and drape shall hang in uniform pleats.

.7	Care to be taken with textured ceilings and/or painted surfaces when mounting tracks.

.8	Allow for a 150 mm stackback clearance near doors.

END OF SECTION