Section Cover Page

Section 00825 2004-12-15 Metric Usage

Use this Section on all projects, except where deemed inappropriate due to the size or nature of the project.  It will normally not be appropriate for use on small projects.

This Master Specification Section contains:

.1	This Cover Sheet

.2	Specification Section Text:

.1	Metric Policy

.2	General Usage

.3	Structural Steel

.4	Pipes and Tubes

.5	Flat Metal Products

The department’s policy requires that Contract Documents (drawings, specifications, schedules, etc.) for all projects be produced using SI metric measurement exclusively.

Ensure that there is no conflict between this Section and the actual use of metric measurement in the Contract Documents.

Edit Section to suit project requirements.

1.	METRIC POLICY

.1	Contractor shall provide metric Products in the sizes specified in the Contract Documents except where a valid claim can be made that a particular product is not available on the Canadian market.

.2	Claims for exemption from use of metric Products shall be in writing and fully substantiated with supporting documentation.  Promptly submit claims to Minister for consideration and ruling.  Non-metric Products may not be used unless Contractor's claim has been approved in writing by the Minister.

.3	Difficulties caused by Contractor's lack of planning and effort to obtain metric Products which are available on the Canadian market will not be considered sufficient reason for claiming that they cannot be provided.

.4	Claims for additional costs due to provision of specified metric Products will not be considered.

2.	GENERAL USAGE

.1	SI metric units of measurement are used exclusively in the Contract Documents, with possible exceptions in some instances.  In case of conflict between metric and imperial terminology, metric terminology shall govern.

.2	The metric language conforms to CAN/CSA-Z234.1-89, Canadian Metric Practice Guide.

SPEC NOTE:  Delete the following if no pressures are specified in the Contract Documents.

.3	All pressures specified are gauge pressures unless otherwise indicated.

SPEC NOTE:  Delete the following if no structural steel is specified in the Contract Documents.

3.	STRUCTURAL STEEL

.1	Refer to "Metric Structural Steel Design Data" published by Canadian Institute of Steel Construction.

SPEC NOTE:  Delete the following if no pipes or tubes are specified in the Contract Documents.

4.	PIPES AND TUBES

.1	Equivalent nominal diameters for pipes and tubes are given in Table 1.

| TABLE 1 |  |  |  |
| --- | --- | --- | --- |
| in mm | in. mm | in. mm | in. mm |
| 1/4 6 | 2 1/2 65 | 15 375 | 36 900 |
| 1/16 8 | 3 80 | 16 400 | 39 975 |
| 3/8 10 | 3 1/2 90 | 18 450 | 40 1000 |
| 1/2 13 | 4 100 | 20 500 | 44 1100 |
| 5/8 16 | 5 125 | 21 525 | 48 1200 |
| 3/4 20 | 6 150 | 22 550 | 52 1300 |
| 7/8 22 | 7 175 | pl. 560 | 56 1400 |
| 1 25 | 8 200 | 24 600 | 60 1500 |
| 1 1/8 28 | 9 225 | pl. 630 | 64 1600 |
| 1 1/4 30 | 10 250 | 26 650 | 72 1800 |
| 1 3/8 35 | 11 275 | 27 675 | then by |
| 1 1/2 40 | pl. 280 | 28 700 | multiples |
| 1 5/8 41 | 12 300 | pl. 710 | of 200 mm |
| 1 3/4 44 | pl. 315 | 30 750 | to 4000 mm |
| 2 50 | pl. 355 | 33 825 |  |

SPEC NOTE:  Delete the following if no flat metal products are specified in the Contract Documents.

5.	FLAT METAL PRODUCTS

.1	Thicknesses of flat metal products are generally specified in the Contract Documents using the preferred nominal thicknesses in millimeters as listed in columns 1 and 2 of Table 2, which represent hard conversions. In some instances certain products (eg. roof flashings) may be specified using nominal thicknesses in millimeters as listed in columns 4, 6, 8, and 10 which represent a soft conversion of previously used nominal thicknesses in inches.

.2	Until the industry has completed hard conversion to the preferred metric dimensions, Table 2 may be used as an aid in correlating previously used gauge numbers and their soft converted nominal thickness equivalents with the hard converted preferred nominal thicknesses and vice versa.

.3	Following is an explanatory guide to Table 2:

.1	Columns 1 & 2: Preferred nominal thicknesses stated (first preference and second preference) conform to CAN3-G312.1-75, preferred Metric Dimensions for Flat Metal Products, and represent hard conversions.

.2	Columns 3, 5, 7 and 9: Gauge numbers are no longer used to specify thicknesses, however they are provided here for correlation with previous practice.

.3	Columns 4, 6, 8 and 10: Thickness equivalents stated are nominal and are soft conversions of previously used nominal thicknesses in inches. They do not take into account allowable negative tolerances nor do they represent actual minimum thicknesses.

| TABLE 2 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PREFERRED NOMINAL |  | GAUGES AND NOMINAL THICKNESS EQUIVALENTS |  |  |  |  |  |  |  |  |
| THICKNESSES IN (mm) |  | GALVANIZED STEEL |  | UNCOATED STEEL (HOT & COLD ROLLED) |  | STAINLESS STEEL |  |  | NONFERROUS METAL |  |
| 1st Pref. | 2nd Pref. | GSG | Thick. Equiv.* (mm) | MSG | Thick. Equiv. (mm) | USSG | Thick. Equiv. (mm) | AG or B&SG |  | Thick. Equiv. (mm) |
| 0.30 |  | 32 | 0.3404 | 30 | 0.3048 | 30 | 0.3175 | 28 |  | 0.3200 |
| 0.40 |  | 30 | 0.3988 | 28 | 0.3785 | 28 | 0.3969 | 26 |  | 0.4039 |
| 0.50 |  | 28 | 0.4750 | 26 | 0.4547 | 26 | 0.4763 | 24 |  | 0.5105 |
| 0.60 |  | 26 | 0.5512 | 24 | 0.6071 | 24 | 0.6350 | 22 |  | 0.6452 |
|  | 0.70 | 24 | 0.7010 |  |  |  |  |  |  |  |
| 0.80 |  | 22 | 0.8534 | 22 | 0.7595 | 22 | 0.7938 | 20 |  | 0.8128 |
| 1.0 |  | 20 | 1.006 | 20 | 0.9119 | 20 | 0.9525 | 18 |  | 1.0236 |
| 1.2 |  | 18 | 1.311 | 18 | 1.2141 | 18 | 1.2700 | 16 |  | 1.2903 |
| 1.6 |  | 16 | 1.613 | 16 | 1.5189 | 16 | 1.5875 | 14 |  | 1.6281 |
| 2.0 |  | 14 | 1.994 | 14 | 1.8974 | 14 | 1.9844 | 12 |  | 2.0523 |
| 2.5 |  |  |  | 12 | 2.6568 |  |  | 10 |  | 2.5883 |
| 3.0 |  | 12 | 2.753 |  |  | 12 | 2.7781 |  |  |  |
| 3.5 |  | 10 | 3.510 | 10 | 3.4163 | 10 | 3.5719 | 8 |  | 3.2639 |
| 4.0 |  |  |  | 8 | 4.1758 |  |  | 6 |  | 4.1148 |
| 4.5 |  | 8 | 4.270 |  |  | 8 | 4.3656 |  |  |  |
| Col.1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |  | 10 |

* The thickness equivalents for galvanized steel include both base

steel and zinc coating.

END OF SECTION