Section Cover Page

Section 02821 2000-03-22 Chain Link Fencing

This Master Specification Section contains:

.1	This Cover Sheet

.2	Specification Section Text:

1.	General

1.1	Detail Drawings

1.2	Measurement And Payment

2.	Products

2.1	Standards

2.2	Materials

2.3	Components

3.	Execution

3.1	Installation

.3	Detail Drawings:

02821.01	Gate Set-Back Detail

02821.02	Standard Chain Link Fence

02821.03	Typical Chain Link Pedestrian Gate

02821.04	Typical Chain Link Vehicle Gate

Applicable Standards:

.1	ASTM A53-96	Pipe, Steel, Black and Hot-Dipped, Zinc Coated, Welded and Seamless

.2	CAN2-138.1-96	Fabric for Chain Link Fence

.3	CAN2-138.2-96	Steel Framework for Steel Fence

.4	CAN2-138.3-96	Installation for Chain Link Fence

These reference documents may be available for reference purposes in the Technical Resources and Standards Branch Library, Alberta Infrastructure, 3rd Floor, 6950 - 113 Street, Edmonton; Telephone:  (780) 427-7924.

1.	General

1.1	DETAIL DRAWINGS

.1	Following detail drawings are appended hereto and form part of this Section.

Number	Title

02821.01	Gate Set-Back Detail

02821.02	Standard Chain Link Fence

02821.03	Typical Chain Link Pedestrian Gate

02821.04	Typical Chain Link Vehicle Gate

1.2	MEASUREMENT AND PAYMENT

SPEC NOTE:  Use for Unit Price Contracts only.

.1	Chain link fence installed will be paid by metre length measured along fence line.

.2	Corner and terminal posts will be paid by the unit installed.

.3	Gate leaves and gate posts will be paid by the complete unit installed.

2.	Products

2.1	STANDARDS

.1	Steel Pipe to ASTM A53-96.

.2	Chain Link Fabric to CAN2-138.1-96.

.3	Fence, Chain Link, Frame Work, Zinc-Coated, Steel to CAN2-138.2-96.

.4	Fence, Chain Link Installation to CAN2-138.3-96.

2.2	MATERIALS

.1	Pipe:  Steel butt weld, Schedule 40, hot dip galvanized to 550 g/m2 coating.

.2	Mesh Wire:  [Galvanized steel wire hot-dipped galvanized to 490 g/m2] [Galvanized steel wire hot-dipped galvanized to 90 g/m2 and polyvinylchloride coated to [0.560] [0.380] mm thickness.

.3	Concrete:  [Sulphate resistant] [Normal] Portland cement, 20 MPa @ 28 days, 50 mm to 80 mm slump, 20 mm aggregate, 6% air entrainment.

2.3	COMPONENTS

.1	Line Posts:  [48 mm O.D.,4.05 kg/m] [60 mm O.D., 5.43 kg/m].

.2	Corner, Terminal and Straining posts:

[60 mm O.D., 5.43 kg/m] [73 mm O.D., 8.62 kg/m] [89 mm O.D., 11.28 kg/m].

.3	Gate Posts:  [89 mm O.D., 11.28 kg/m] [114 mm O.D., 16.06  kg/m] [168 mm O.D., 28.23 kg/m].

.4	Top and Brace Rail:  [33 mm O.D., 2.51 kg/m] [42 mm O.D., 3.38 kg/m] plain end, sleeve coupled.

.5	Gate Frame:  42 mm O.D., 3.38 kg/m.  Gate leaves to have horizontal and vertical intermediate brace on gate leaves 3.0 m wide and over.

.6	Post Caps:  Cast aluminum, sized to post diameter, set screw retained.

.7	Extension Arms:  Cast aluminum to accommodate 3 strands of barbed wire, [vertical] [sloped 45°], top strand 300 mm from fence fabric].

.8	Line Post Eye tops:  Cast aluminum.

.9	Rail Ends:  Cast aluminum.

.10	Fittings:  Sleeves, bands, clips, tension bars, fasteners and fittings galvanized steel.

.11	Fabric:  50 mm diamond mesh, interwoven 3.5 mm wire, top selvage twisted tight, bottom selvage knuckle end closed.

.12	Bottom tension Wire:  3.0 mm steel single strand hot-dipped galvanized to 490 g/m2

.13	Barbed Wire:  2.5 mm wire, three strands, four point barbs at 150 mm on centre, zinc coated steel.

.14	Single Gate Hardware:  2 piece [drop] [lift] latch and latch catch.  Gate hinge 180° male and female [hardware for padlock] [chain hold open].

.15	Double Gate Hardware:  Gate centre rest, [2 piece [drop] [lift] latch and latch catch with foot bolt] [3 piece drop latch and latch catch with drop bolt.  Gate hinge 180° male and female.  [Hardware for padlock. [[chain hold open] [gate keeper post and ground latch stop]].

.16	Sliding Gates:  48 mm O.D., 4.05 kg/m for top, bottom and upright members and 42 mm O.D., 3.38 kg/m for horizontal and diagonal members.  Free-operating roller assembly.

3.	Execution

3.1	INSTALLATION

.1	Install to alignment specified, line posts, corner posts, gate posts and top rails to provide rigid structure for [   ] [1.2 m] [1.8 m] high fabric and gates.

.2	Maximum spacing of posts:  3.0 m on centre

.3	Install line and corner posts plumb.

.4	Set posts in cylindrical cast-in-place concrete footings sized as follows:

| Location | Depth | Diameter |
| --- | --- | --- |
| Fence height up to 1.2 m: Line posts Gate and corner posts | 900 mm 1000 mm | 250 mm 300 mm |
| Fence height up 1.2 to 1.8 m: Line posts Gate and corner posts | 1000 mm 1200 mm | 250 mm 300 mm |

.5	Set posts to within 150 mm from bottom of concrete footing.

.6	Set top of concrete footing 50 mm above finished grade.  Slope top of footing to ensure water run off.

.7	Position bottom of fabric 50 mm above finished grade with tension wire stretched taut between posts.

.8	Align top of posts to ensure that top rail varies gradually with changes in ground elevations.

.9	Pass top rail through line post tops to form continuous bracing.  Install 150 mm long couplings mid-span at pipe ends.

.10	Brace each gate and corner post back to adjacent line post with horizontal centre brace rail.  Install brace rail, one bay from corner and gate posts.

.11	Install 10 mm steel truss rod and truss tightener diagonally from top of gate post back to adjacent line post.

SPEC NOTE:  Specify truss rod and tightener where gate leaves are 3.0 m wide and over or for gate leaves for 1.8 m high and over fence.

.11	Fasten fabric to top rail, line posts, braces and bottom tension wire with 3.5 mm wire ties maximum 500 mm centres.

.12	Attach fabric to corner and gate posts with tension bars and tension bar clips.  Stretch fabric between posts at intervals of 30.0 m maximum.

.13	Install straining post at 90 m approximately.

.14	Install 3 strands of barbed wire on arms, tensioned and secured.  Slope extension arms for barb wire [inward] [outward.

.15	Install gates of sizes shown using fabric [and vertical extension arms] to match fence.  Install 3 hinges per leaf and hardware specified.

.16	Provide concrete centre rest and drop bolt retainers at centre of double gate openings.

.17	Install sway brace units on gate posts where gate leaves are 4.9 m and over in width.

END OF SECTION