Section Cover Page

Section 07461 1999-09-23 Wood Siding

This Master Specification Section contains:

.1	This Cover Sheet

.2	Specification Section Text:

1.	General

1.1	Related Work Specified in Other Sections

1.2	Product Options and Substitutions

1.3	Samples

2.	Products

2.1	Materials

2.2	Related Accessories

3.	Execution

3.1	Installation

Applicable Standards

.1	CSA A123.3-M1979(R1992)	Asphalt or Tar Saturated Roofing Felt

.2	CSA O121-M1978	Douglas Fir Plywood

.3	CSA O151-M1978	Canadian Softwood Plywood

.4	CSA O153-M1980	Poplar Plywood

These reference documents may be available for reference purposes in the Technical Resources and Standards Branch Library, Alberta Infrastructure, 3rd Floor, 6950 - 113 Street, Edmonton; Telephone: (780) 427-7924.

1.	General

1.1	RELATED WORK SPECIFIED IN OTHER SECTIONS

.1	Wall sheathing backup:	Section 06100.

.2	Sheet metal roof flashings:	Section 07625.

.3	Caulking and sealants:	Section 07920.

1.2	PRODUCT OPTIONS AND SUBSTITUTIONS

.1	Refer to Division 1 for requirements pertaining to product options and substitutions.

1.3	SAMPLES

.1	Submit duplicate samples in accordance with Division 1.

.2	Samples to be representative of material, finish, colour and profile of specified work.

2.	Products

2.1	MATERIALS

.1	Board Siding:  [horizontal bevel] [vertical [tongue and groove] [channel cut] [board and batten]], [western red cedar] [          ] species, [          ] surface finish, [     mm x     mm] nominal board size.

.2	Sheet Siding:  Panels of 1200 mm x 2400 mm nominal x [     mm] thick, of [douglas fir plywood to CSA 0121-M1978] [western softwood plywood to CSA 0151-M1978] [popular plywood to CSA 0153-M1980] [western red cedar plywood], vertically grooved on [200 mm] [     mm] centres, [striated] [rough sawn] [unsanded] [sanded] surface finish, prime coated in [     ] colour stain, shiplapped edges.

.3	Shingle Siding:  western red cedar, No. [1] [2] [3] [4] quality, [smooth sanded] [handsplit and resawn] [taper split] [straight split] [      ] surface finish, [    mm x     mm to     mm] thick.

.4	Accessories:  exposed trim, closures, cap pieces of same material and colour as siding.

2.2	RELATED ACCESSORIES

.1	Building Paper:  [No. 15 type asphalt felt to CSA A123.3-M1979] [No. 25 type asphalt felt to CSA A123.3-M1979].

.2	Nails:  [zinc coated steel] [aluminum] [copper], sized as required, with [sinker head] [casing] [finishing] [sinker head ring shanked] nail.

.3	Flashings:  0.60 mm galvanized steel, 0.80 mm aluminum sheet, brake formed to profile required, in maximum lengths.

3.	Execution

3.1	INSTALLATION

.1	Confirm acceptability of wall sheathing for soundness, measurement and flatness.

.2	Install building paper horizontally by [stapling] [nailing], lapping edges, 75 mm [one layer] [two layers.]

.3	Install sill flashings, wood starter strips, inside corner flashings, edgings and flashings over openings.

.4	Install siding with edges square and vertical, tight to edges and openings, lapping over metal flashings and trim.  [Shiplap edges tight and secure.]  Nail to sheathing to an orderly pattern at 300 mm O.C. at intermediate and 150 mm O.C. at edge locations.  Miter any horizontal joints at 45° tight.

.5	Install shingles to [single] [double] course method to ensure [double] [triple] thickness of shingle at any given point.  [For double coursing set outer course shingle 12 mm lower than under course grade shingle.]  At external corners install shingle over mitered metal flashing. Install and secure shingles to standards established by the "Red Cedar Shingle and Handsplit Shake Bureau".

SPEC NOTE:  Select either or both clauses .4 and .5 above.

.6	Install exterior corner cover strips, closures, and trim.

END OF SECTION