Section Cover Page

Section 07465 1999-09-23 Metal Siding

This Master Specification Section contains:

.1	This Cover Sheet

.2	Specification Section Text:

1.	General

1.1	Related Work Specified in Other Sections

1.2	Product Options and Substitutions

1.3	Reference Standards

1.4	Design

1.5	Samples

1.6	Shop Drawings

1.7	Delivery/Storage

2.	Products

2.1	Materials

2.2	Components

3.	Execution

3.1	Installation

Applicable Standards:

.1	CGSB 19-GP-5M	Sealing Compound, One Component, Acrylic Base, Solvent Curing

.2	CAN/CGSB-19.18-M87	Sealing Compound, One-Component, Silicone Base, Solvent Curing

.3	CAN/CGSB-19.24-M90	Multicomponent, Chemical-Curing Sealing Compound

.4	CAN/CGSB-93.1-M85	Sheet, Aluminum Alloy, Prefinished, Residential

.5	CAN/CGSB-93.2-M91	Prefinished Aluminum Siding, Soffits and Fascia, for Residential Use

.6	CAN/CGSB-93.3-M91	Prefinished Galvanized and Aluminum-Zinc Alloy Steel for Residential Use

.7	CAN/CGSB-93.4-92	Galvanized and Aluminum-Zinc Alloy Coated Steel Siding, Soffits and Fascia, Prefinished, Residential

.8	CAN/CGSB-93.5-92	Installation of Metal Residential Siding, Soffits and Fascia

.9	CSA A123.3-M1979 (R1992)	Asphalt or Tar Saturated Roofing Felt

These reference documents may be available for reference purposes in the Technical Resources and Standards Branch Library, Alberta Infrastructure, 3rd Floor, 6950 - 113 Street, Edmonton; Telephone: (780) 427-7924.

1.	General

1.1	RELATED WORK SPECIFIED IN OTHER SECTIONS

.1	Wall sheathing backup:	Section 06100.

.2	Sheet metal roof flashings:	Section 07625.

1.2	PRODUCT OPTIONS AND SUBSTITUTIONS

.1	Refer to Division 1 for requirements pertaining to product options and substitutions.

1.3	REFERENCE STANDARDS

.1	Materials standards to [CAN/CGSB 93.1-M85] [CAN/CGSB 93.2-M91] [CAN/CGSB 93.3-M91] [CAN/CGSB 93.4-92]

.2	Perform installation to CAN/CGSB-93.5-92.

SPEC NOTE:  Include 1.4 and 1.5 only for large involved projects.

1.4	DESIGN

.1	Siding shall be designed to withstand positive wind load of [     ] 1 kPa and negative load of [     ] 0.6 kPa at a maximum allowable deflection of [     ] [1/180] of span between attachments.

.2	Indicate test data supporting the above requirements on shop drawing submission.

1.5	SAMPLES

.1	Submit duplicate samples in accordance with Division 1.

.2	Samples to be representative of material, finish, colour and profile of specified work.

1.6	SHOP DRAWINGS

.1	Submit shop drawings in accordance with Division 1.

.2	Clearly indicate dimensions, siding profiles, attachment methods, schedule of wall elevations, trim and closure pieces, [soffits,] [fascia,] and related work.

1.7	DELIVERY/STORAGE

.1	Protect materials from damage by weather or workmen.

.2	Ensure packaging of prefinished materials permits ventilation.

2.	Products

2.1	MATERIALS

.1	Exterior Sheet:  aluminum sheet, to CAN/CGSB-93.1-M85 type [1] [2] class [F1S] [F2S], [plain] [patterned] surface, [0.48 mm] [0.61 mm] base metal, factory prefinished to colour selected by Minister.

SPEC NOTE:  Refer to CAN/CGSB-93.3-M91 Table 1 for base metal thickness.

.2	Exterior Sheet:  steel sheet to CAN/CGSB-93.3-M91 class [F1S] [F2S], 0.399 mm base metal including Z275 zinc coating, factory prefinished to colour selected by Minister.

.3	Building Paper:  [#15 asphalt felt to CSA A123.3-M1979]

.4	Sealant:  [one component acrylic to CGSB 19-GP-5M [two component polysulphide to CAN/CGSB-19.24-M90 [one component silicone to CAN/CGSB-19.18-M87.]

2.2	COMPONENTS

.1	Aluminum Siding:  to CAN/CGSB-93.2-M91 Type [HU-horizontal siding-unbacked] [HB-horizontal siding-backed] [VU-vertical siding-unbacked] [SF-soffit and fascia facings and exposed tri-munbacked] Class [1-plain] [2-patterned.]

.2	Steel Siding:  to CAN/CGSB-93.4-92 [horizontal siding] [vertical siding] [soffit and fascia facings and exposed trim] type, [plain] [patterned] class, [with] [without] backing.

.3	Siding:  [     mm profile, [     mm] deep, preformed male and female lip joints, nailing strip prepunched, [backing material glued to back surface of face sheet with all weather contact cement.]

.4	Soffit:  of flat exterior sheet, 'V' crimped for stiffness, [vented 0.1 m2 of opening for every 30 m2 of building area] [perforated with elongated slits and small perforations] [insect screen cover at vents], of same material and colour as siding.

.5	Accessories:  exposed trim, closures, cap pieces, etc. of same material and colour as siding.

3.	Execution

3.1	INSTALLATION

.1	Confirm acceptability of wall sheathing of soundness, measurement and flatness.

.2	Install to CAN/CGSB-93.5-92 and as specified herein.

.3	Install building paper horizontally by [stapling] [nailing] lapping edges 75 mm, one layer.

.4	Install sill flashings, starter strips, inside corners, edgings, and soffits.

.5	Install siding [and attachments] [sequentially] [from sill up], to manufacturer/fabricators written instructions.

.6	Install exterior corners, fillers and closure strips with carefully formed and profiled work.  Install with concealed fasteners.

.7	Maintain joints in exterior sheets, true to line, tight fitting.

.8	Caulk joints, seams and junctions with dissimilar materials, with specified sealant.  Refer to Section 07910 for caulking technique and workmanship.

.9	Provide all components including drip and cap flashings, screws and fasteners as required to complete installation.

END OF SECTION