Section Cover Page

Section 08120 1999-09-27 Aluminum Doors and Frames

This Master Specification Section contains:

.1	This Cover Sheet

.2	Specification Section Text:

1.	General

1.1	Related Work Specified in Other Sections

1.2	Product Options and Substitutions

1.3	Design

1.4	Shop Drawings

2.	Products

2.1	Materials

2.2	Finishes

2.3	Hardware

2.4	Fabrication (General)

2.5	Fabrication (Aluminum Doors)

2.6	Fabrication (Aluminum Frames)

3.	Execution

3.1	Installation

3.2	Glazing

3.3	Caulking

3.4	Cleaning

3.5	Hardware Schedule

Applicable Standards:

.1	CAN/CGSB-1.40-M89	Primer, Structural Steel, Oil Alkyd Type

.2	CAN/CGSB-1.108-M89	Bituminous Solvent Type Paint

.3	CAN/CSA-G40.21-M92	Structural Quality Steels

These reference documents may be available for reference purposes in the Technical Resources and Standards Branch Library, Alberta Infrastructure, 3rd Floor, 6950 - 113 Street, Edmonton; Telephone: (780) 427-7924.

1.	General

1.1	RELATED WORK SPECIFIED IN OTHER SECTIONS

.1	Steel support framing above ceiling/soffit:	Section 05501.

.2	Rough wood blocking:	Section 06100.

.3	Insulation:	Section 07213.

.4	Caulking and sealants between frames and other building components:	Section 07920.

.5	Metal door frames and screens:	Section 08111.

.6	Hollow metal doors:	Section 08115.

.7	Aluminum windows:	Section 08520.

.8	Glass and glazing:	Section 08801.

1.2	PRODUCT OPTIONS AND SUBSTITUTIONS

.1	Refer to Division 1 for requirements pertaining to product options and substitutions.

1.3	DESIGN

.1	Design members to withstand, within acceptable deflection limitations, their own weight, the weight of glass, and the minimum design loads due to the pressure and suction of wind as calculated in accordance with Alberta Building Code.

.2	Make provision to drain, to the exterior, any water entering at joints and/or condensation occurring within the wall construction.

.3	Design assembly to accommodate expansion and contraction when subjected to minimum and maximum surface temperature of -35°C to +75°C.

.4	Deflection limit for mullions shall be governed by flexure limit of glass unit to permissible maximum by glass manufacturer and calculated with a safety factor of 2.5 with full recovery of glazing compounds or [     ] [1/175] maximum deflection of unsupported length; whichever is less.

1.4	SHOP DRAWINGS

.1	Submit shop drawings in accordance with Division 1.

.2	Clearly indicate, by large scale details, all perimeter conditions of construction, mullion/muntin details, all components of the assembly, anchorage, sealing techniques, tolerances of openings, elevations, all materials, hardware and finishes.

2.	Products

2.1	MATERIALS

.1	Aluminum extrusions:  Aluminum Association alloy [AA6063-T5] [AA6006-T5].

SPEC NOTE:  Specify alloy AA6006-75 when hardcoat finish is selected.

.2	Sheet aluminum:  Aluminum Association alloy AA1100 [anodizing quality].

.3	Steel reinforcements:  to CAN/CSA-G40.21-M92.

.4	Fasteners:  aluminum, stainless steel type 316, or cadium plated steel, finished to match adjacent material.

.5	Steel primer:  to CAN/CGSB-1.40-M89.

.6	Bituminous paint:  to CAN/CGSB-1.108-M89.

.7	Weatherstripping:  waterproof, rot-proof pile fibre 4 mm high x 6 mm wide in neoprene backing of flexible vinyl.

.8	Sills:  sloped extruded aluminum sill sections including upturned end stops, chairs, anchors, splice plates; finish same as aluminum framing.

.9	Insulated panels:  outer facing of [     ] mm sheet [aluminum] [galvanized sheet steel], inner facing of [     ] mm galvanized sheet steel, laminated to core material of [     ] insulation, form edges [closed seam] [to lip for glazing into wall system, provide for condensation drainage to wall system weep drainage].

.10	Glass and glazing materials:  in accordance with Section 08801.

.11	Sealants:  in accordance with Section 07920, paragraph [     ], [      ] colour [selected by Minister].

SPEC NOTE:  Refer to Section 07920 and select appropriate sealant material and insert paragraph number and colour as required in blank spaces.

2.2	FINISHES

.1	Finish on exposed aluminum surfaces shall be [mill finish] [anodized to Alcan [Analok] [Hardcolour] [[6] [10] [15] [20] [25] micrometres] thick, finish designation [A22] [A32] [A41] [A42] [A4x], clear [light bronze] [medium bronze], [dark bronze] [black] colour] [enamelled to Alcan [     ] with thermosetting acrylic enamel, colour selected by Minister].

.2	Shop apply strippable protective [coating] [covering] to finished aluminum surfaces until completion of the work.

.3	Paint ungalvanized steel clips, supports and reinforcing steel with steel primer or bituminous paint.

.4	Finish on insulated panels shall be [     ], colour selected by  Minister.  [Interior surface of panel may be left natural].

2.3	HARDWARE

SPEC NOTE:  Schedule items as per paragraph 3.5.

SPEC NOTE:  For automatic doors use centre pivot hinges.

.1	Butts:  [1 1/2] [2] pair per door leaf, [114 mm x 102 mm] [114 mm x 114 mm] [127 mm x 114 mm], FBB 199, NRP, [     ] finish.

Pivot hinges:  [offset] [centre] pivots, self-lubricating bronze bearings, hardened steel pins, adjusting pivot with locking screw, [mortised intermediate pivot hinge], finish shall match door frame.

.2	Cylinder:  [supplied under Section 08700] [mortised cam type, [master keyed to building keying system], 5 pin type, finish same as door, [thumb turn], mount 1520 mm above floor.

.3	Maximum security lock:  5 ply steel bolt, 35 mm throw, anti-saw roll pins, to accept mortised cylinder.

.4	Push:  aluminum bar with push plate similar to Kawneer style [     ], mount 1140 mm above floor.

.5	Pull:  extruded aluminum shape similar to Kawneer style [     ], mount 1140 mm above floor.

.6	Panic:  [concealed] [surface] rim device, Russwin [     ] model, [roller strike], mount 1025 mm above floor.

.7	Threshold:  extruded aluminum, ribbed, full width of opening, [as required to cover floor mounted closer].

.8	Closer:  [single] [double] acting, overhead, [surface] [concealed] mounted, rectangular slim profile, LCN [     ] model [including support bracket], [     ] finish, adjustable closing and latch speeds.

.9	Floor closer:  [offset] [centre] pivot type, [single] [double] acting, hydraulic action, [105° hold open], floor cover plate [flush with threshold], metal concrete form box, adjustable closing and latch speeds.

.10	Holder:  overhead [surface] [concealed] type, adjustable check for [     °] opening, Glynn-Johnson Model [     ], [     ] finish.

2.4	FABRICATION (GENERAL)

.1	Provide minimum 20 mm bite for factory-sealed double glazed units.  Ensure sufficient bite on all other glass as required for live and dead loads.

.2	Make allowances for deflection of structure to ensure that structural loads are not transmitted to aluminum work.

.3	Provide structural steel internal reinforcement as required to achieve specified criteria for frame stability.

.4	Manufacturers' nameplates on doors or frames are not acceptable.

.5	Accurately join and fit corners of doors and frames to flush hairline joints [and weld along concealed lines of contact].

.6	Accurately form cut-outs, recesses, mortising or milling required for finishing hardware in accordance with templates supplied and reinforce with galvanized or stainless steel plates.

2.5	FABRICATION (ALUMINUM DOORS)

.1	Construct doors of porthole extrusions with 3.0 mm wall thickness.

.2	Door stiles nominal [     ] mm wide ± 6 mm.

.3	Top rail nominal [     ] mm wide ± 6 mm.

.4	Bottom rail nominal [     ] mm wide ± 6 mm.

.5	Reinforce mechanically-joined corners of doors by welding and spigotting or one piece cast aluminum angle to produce sturdy door unit resistant to wracking.

.6	Provide weatherstripping on two sides and head of exterior doors and sweep strip full width of bottom rail.  Construct to permit ready placement of weatherstripping on installed doors without special tools.

.7	Provide glazing stops of interlocking snap-in type for dry glazing.  Design and make exterior stops tamperproof type.

2.6	FABRICATION (ALUMINUM FRAMES)

.1	Construct frames of aluminum extrusions, [     mm x     mm] size, 3.0 mm thickness [flush] [applied] stops, [screw applied] [snap-on], [thermally broken sections], [split mullion].

.2	Vertical mullions shall be continuous.  Sills shall be one piece or flush jointed on mullion centreline with alignment spline.

.3	Provide for adequate clearance and shim space at perimeter of opening.

.4	Provide weep holes in horizontal members of exterior frames and screens.  Drain weepholes to exterior.

3.	Execution

3.1	INSTALLATION

.1	Erect and secure aluminum work plumb, square and level, free from warp, twist and superimposed loads.

.2	Secure work adequately and accurately to structure in designed location, in manner not restricting thermal movement.  Lock settings after alignment.

.3	Backpaint aluminum in contact with dissimilar metals and cementitious materials; with bituminous paint.

.4	Protect aluminum and glass surfaces until final cleaning.

.5	Completed installation shall be free from objectionable noise, rattles, wind whistles and noise due to thermal movement.

.6	Fit finish hardware neatly, accurately; make plumb and level with proper clearance for correct operation.

3.2	GLAZING

.1	Glaze aluminum doors, screens and sidelights using [     ] glass in accordance with Section 08801.

SPEC NOTE:  If drawings or schedules do not indicate types and thicknesses of glass required for various locations, detail the requirements here.  Ensure types of glass required in this Section are included in Section 08801.

3.3	CAULKING

.1	Install sealant and related materials for perimeter of frame and between members of aluminum work in accordance with Section 07920 to provide a weatherproof installation.

.2	Ensure continuity of air/vapour barrier with adjacent construction.

3.4	CLEANING

.1	Wash down exposed interior metal surfaces using a solution of mild domestic detergent in warm water, applied with soft clean wiping cloths.

.2	Clean exposed exterior non-metal surfaces as recommended by the manufacturer of the material.

.3	Wipe interior surfaces clean when construction is completed.  Clean and polish finish hardware.

3.5	HARDWARE SCHEDULE

.1	Provide and install hardware to the following schedule:

SPEC NOTE:  Set out a hardware schedule listing door numbers down left column and set out column headings as for specified items:  Room # to Room #, handling, single or double leaf, hinges, cylinder, push/pull, panic, threshold, closer, holders, weatherstripping.

END OF SECTION