Section Cover Page

Section 08353 1999-09-27 Folding Accordion Doors

This Master Specification Section contains:

.1	This Cover Sheet

.2	Specification Section Text:

1.	General

1.1	Related Work Specified in Other Sections

1.2	Samples

1.3	Shop Drawings

2.	Products

2.1	Materials/Components

3.	Execution

3.1	Erection

1.	General

1.1	RELATED WORK SPECIFIED IN OTHER SECTIONS

.1	Steel support framing:	Section [05120][05501].

.2	Wood blocking:	Section 06100.

.3	Wood finish trim:	Section 06200.

.4	Acoustic ceiling:	Section [   ].

1.2	SAMPLES

.1	Submit 300 x 300 mm sample of vinyl fabric covering for approval.

1.3	SHOP DRAWINGS

.1	Submit shop drawings in accordance with Division 1.

.2	Indicate opening sizes, head, jamb and floor conditions, fabric characteristics, hardware and attachments.

2.	Products

2.1	MATERIALS/COMPONENTS

.1	Folding Partition:  accordion fold pantograph action, internal galvanized 1.63 mm steel hinge plates, 5 mm Dia. rods, [limiting mechanism] [limit chains], [stacking straps and snaps, ball bearing trolleys at every second hinge], [acoustic seals at top, bottom and both jambs], internal air release, [sound absorbent liner.]

.2	Vinyl Fabric Covering:  [[0.5] [0.75] [     ] kg/m2] vinyl fabric covering of [    kg/m2] vinyl coating laminated to [     kg/m2] mildew resistant backing fabric, [with ULC fire hazard classification maximum ratings of flame spread 25, fuel contributed 10 and smoke developed 50,] [clear plastic finish coating for increased washability], of [        ] texture, colour as selected by Minister.

.3	Track:  [formed steel] [extruded aluminum], [32 mm x 32 mm] [41 mm x 45 mm] designed to support partition live and dead loads; including steel sub-channel and track connectors, [white enamelled ceiling closure] [white enamelled ceiling guard.]

.4	Accessories:  aluminum jamb moulding with integral resilient seal, and all fitments required.

.5	Hardware:  latching door handles of cast steel satin chrome finish, [lock cylinder master keyed to building keying system] [pull bars.]

3.	Execution

3.1	ERECTION

.1	Install folding door to manufacturer's printed instructions, to a complete and operating installation.

.2	Door shall glide smoothly from stacked to drawn position without undue effort.

.3	Coordinate installation of components with related and adjacent work.

.4	Clean down fabric surfaces and final adjust.

END OF SECTION