Section Cover Page

Section 08831 1999-09-27 Mirror Glass

Use this section to specify unframed fully-silvered glass mirrors for mounting on walls.

Specify framed and backed mirror units, preassembled in factory, in Division 10.

Specify light and heat reflecting glass and transparent, one way mirrored glass in Section 08801 - Glass and Glazing General Requirements.

Specify mirror tiles in a special tile section in Division 9.

Specify packaged mirror-door units ordinarily used for closet doors in a Division 8 door section.

This Master Specification Section contains:

.1	This Cover Sheet

.2	Specification Section Text:

1.	General

1.1	Section Includes

1.2	Related Sections

1.3	Delivery, Storage and Handling

2.	Products

2.1	Mirror Glass

2.2	Installation Materials

3.	Execution

3.1	Installation

3.2	Cleaning

3.3	Protection

3.4	Mirror Schedule

Applicable Standards

.1	CAN/CGSB-12.5-M86	Mirrors, Silvered

These reference documents may be available for reference purposes in the Technical Resources and Standards Branch Library, Alberta Infrastructure, 3rd Floor, 6950 - 113 Street, Edmonton; Telephone:  (780) 427-7924.

1.	General

1.1	SECTION INCLUDES

.1	This section includes requirements for supply and installation of wall mounted, unframed, fully silvered glass mirror units.

1.2	RELATED SECTIONS

.1	Glass and Glazing General Requirements:	Section 08801.

.2	Light and heat reflecting glass:	Section 08801.

.3	Transparent, one way mirrored glass:	Section 08801.

.4	Glazing:	Section 08810.

.5	Packaged mirror-door units:	Section [08  ].

.6	Mirror tiles:	Section [09  ].

.7	Metal framed mirror units:	Section [10  ].

1.3	DELIVERY, STORAGE AND HANDLING

.1	Comply with requirements of Section 08801.

2.	Products

2.1	MIRROR GLASS

.1	Mirrors, Silvered:  to CAN/CGSB-12.5-M86 and as follows:

.1	Type:  [1A - Float glass, for normal use] [1B - Float glass for high humidity use] [3A - Float glass, tempered] [3B - Float glass, laminated].

.2	Tint:  [Clear] [Bronze] [      ].

.3	Edges:  [Flat polished edge] [Pencil polished edge] [Beveled edge of    mm width].  Seal edges to prevent chemical or atmospheric penetration of backing.

.4	Decorative Treatment:  [Acid etch] [Engrave] [Sandblast] decorative patterns on mirror glass as indicated on drawings.

2.2	INSTALLATION MATERIALS

.1	Mirror Mastic:  Adhesive setting compound produced specifically for setting of mirrors by spot application, compatible with glass coating and as recommended by mirror manufacturer.

3.	Execution

3.1	INSTALLATION

.1	Wall Mounted Mirror Installation, Generally:

.1	Install mirrors in accordance with mirror manufacturer's instructions.

.2	Mount mirrors in-place to avoid distorting reflected images.

.2	Mastic Installation:

.1	Inspect surface over which mirror is to be mounted.  Comply with mastic manufacturer's installation directions for preparation of mounting surface including coating with mastic manufacturer's special bond coating where applicable.

.2	Apply mastic in spots to comply with mastic manufacturers directions for coverage and to allow air circulation between back of mirror and mounting surface.

.3	Hardware Installation:

.1	Provide permanent means of structural support at bottom edge of mirror.

.2	Attach mirror hardware securely to mounting surface with mechanical fasteners with anchors or inserts.

.3	Place a felt or plastic pad between mirror and metal or plastic clips to prevent spalling of mirror edges.

.4	Mirror Installation:

.1	After mastic is applied, align mirror and press into place while at the same time maintaining a minimum air space of 4 mm for air circulation between back of mirror and mounting surface.

.2	Install clips along top of mirror during mastic curing period.

3.2	CLEANING

.1	Wash mirrors using water or glass cleaners free from substances capable of damaging mirror edges or backing.

3.3	PROTECTION

.1	Protect mirror glass from breakage and contaminating substances resulting from construction operations.

.2	Do not permit edges of mirror to be exposed to standing water.

.3	Maintain environmental conditions which will prevent mirror from being exposed to moisture from condensation or other sources for continuous periods of time.

3.4	MIRROR SCHEDULE

SPEC NOTE:  If more than one mirror type, tint, thickness, etc. is required, schedule locations for each.

| Mirror Type | Tint | Thickness | Location |
| --- | --- | --- | --- |
| [1A] [1B] [3A] [3B] | [ ] | [6 mm][ ] | [ ] |
| [ ] | [ ] | [ ] | [ ] |
| [ ] | [ ] | [ ] | [ ] |

END OF SECTION