Section Cover Page

Section 09641 2000-04-14 Wood Parquet Flooring

Use this section to specify requirements for adhesive applied unfinished or prefinished wood parquet flooring including slat type parquet, unit wood blocks and plastic impregnated blocks.

This Master Specification Section contains:

1.	This Cover Sheet

2.	Data Sheet - General

3.	Specification Section Text:

1.	General

1.1	Related Sections

1.2	Samples

1.3	Shop Drawings

1.4	Maintenance Data

1.5	Maintenance Materials

1.6	Delivery and Storage

1.7	Environmental Requirements

2.	Products

2.1	Wood Subfloor Components

2.2	Wood Parquet

2.3	Accessories

2.4	Finishing Materials

2.5	Fabrication

3.	Execution

3.1	Preparation

3.2	Installation, Wood Subfloor with Sleepers

3.3	Installation, Plywood Subfloor without Sleepers

3.4	Adhesive Application

3.5	Wood Parquet Flooring

3.6	Thresholds

3.7	Sanding and Finishing

Applicable Standards:

.1	CAN/CGSB-1.36-M90	General Purpose Interior Varnish

.2	CAN/CGSB-1.175-M89	Polyurethane Interior Coating, Oil Modified, Clear, Gloss and Satin

.3	CAN/CGSB-25.2-92	Paste Floor Wax

.4	CAN/CGSB-51.34-M86	Vapour Barrier, Polyethylene Sheet for Use in Building Construction

.5	CAN/CGSB-71.27-M88	Adhesive for Bonding Parquet Flooring to Wood and Cured Concrete

.6	CAN/CSA-O80.15-M89	Wood Preservation

.7	CSA O151-M1978	Canadian Softwood Plywood

These reference documents may be available for reference purposes in the Technical Resources and Standards Branch Library, Alberta Infrastructure, 3rd Floor, 6950 - 113 Street, Edmonton; Telephone: (780) 427-7924.

Parquet flooring may be applied over concrete with or without a wood subfloor.  Wood subfloors may be 19 mm plywood directly fastened to concrete, or plywood on wood sleepers.  Wood subfloors on sleepers provide greater cushioning and are recommended where comfort in use is a primary consideration.  Additional cushioning may also be provided by means of rubber pads applied to the underside of plywood subfloors or the underside of wood sleepers.

Consult with manufacturers prior to designing parquet that is to be applied directly to concrete below grade, or on grade located on hillsides.  Subject to manufacturers' approvals, specify liquid-applied waterproofing material compatible with permanently resilient parquet flooring adhesive recommended by flooring manufacturer.  Coordinate with Section 03300 - Cast-in-Place Concrete.  This method may provide adequate protection against moisture and an integral bond with concrete substrate.

Parquet supplied with metal splines is said to impart a slight curvature to parquet and therefore requires thorough rolling after application to ensure maximum adhesive contact.

Plastic impregnated parquet flooring is recommended to greatly reduce wear in heavy traffic areas and for greater resistance to point loading, e.g. high-heel traffic.  While dimensionally more stable, it is not considered to be more moisture resistant than unimpregnated parquet.

1.	General

SPEC NOTE:  Specify wood base associated with parquet flooring in Finish Carpentry - Section 06200 if standard manufactured item.  Specify wood base in Architectural Woodwork - Section 06400 if custom millwork item.

1.1	RELATED SECTIONS

.1	Wood base:	Section [06200] [06400].

1.2	SAMPLES

.1	Comply with requirements of Division 1.

.2	Provide duplicate [     ] mm x [     ] mm samples of wood parquet, finished as specified.

SPEC NOTE:  Determine size of samples based on type and pattern of flooring.

1.3	SHOP DRAWINGS

.1	Comply with requirements of Division 1.

SPEC NOTE:  Specify when pattern layout in various floor areas is not clearly indicated.  Greater control is achieved by clearly detailing requirements on tender documents.

.2	Clearly indicate area of work, placement and orientation of patterns, borders, cuts at edges, entrances, alcoves, change of levels, and other special conditions affecting layout.

.3	Submit product data in duplicate for parquet, adhesives and finishes.

1.4	MAINTENANCE DATA

.1	Comply with requirements of Division 1.

.2	Provide maintenance data for materials and floor finishes.

1.5	MAINTENANCE MATERIALS

.1	Provide [     ] m2 of each type and pattern used on this project.

SPEC NOTE:  On larger projects 1% of installed material area could be required for maintenance purposes.

.2	Include installation and application instructions.

.3	Store where directed.

.4	Clearly identify each box/carton.

1.6	DELIVERY AND STORAGE

.1	Do not deliver material until "wet work" such as concrete and plastering have been completed and allowed to dry as required to provide acceptable substrate moisture content and room relative humidity range.

.2	Store wood and parquet flooring, uncovered, in area of installation meeting environmental requirements, for minimum of 5 days prior to commencing work.

1.7	ENVIRONMENTAL REQUIREMENTS

.1	Maintain ambient temperature between 18°C and 21°C and design relative humidity for 72 hours before installation.

SPEC NOTE:  Include following clause for work in new facilities.  Delete if work to be renovation to an existing occupied facility.

.2	Do not proceed with installation until areas are enclosed, permanent mechanical systems are functioning satisfactorily, and interior relative humidity and temperature meet design requirements.

.3	Maintain environmental requirements after installation completed.

2.	Products

2.1	WOOD SUBFLOOR COMPONENTS

SPEC NOTE:  Delete this article when parquet flooring is to be adhered directly to prepared concrete floor.  Edit article to suit type of subfloor.

.1	Moisture Protection Membrane:  polyethylene film to CAN/CGSB-51.34-M86, 150 micrometres thick.

.2	Sleepers:  38 mm x 64 mm x 1200 mm long kiln dried [spruce] [pine] [fir] [hemlock], preservative pressure treated to CAN/CSA-080.15-M89.

.3	Cork Spacers:  as recommended by flooring manufacturer.

SPEC NOTE:  Specify cork spacers when subfloor sleepers are not fastened to structural floor.

.4	Fasteners:  hot-dip galvanized nails suitable for substrates.

.5	Sheathing plywood:  to CSA O151-M1978, 19 mm thick, tongue and groove edges, sheathing grade.

2.2	WOOD PARQUET

SPEC NOTE:  Ensure drawings indicate selected parquet patterns.

.1	Type:  [slat parquet] [solid unit blocks] [plastic impregnated blocks].

.2	Species:  kiln dried [oak] [hard maple] [beech] [birch] [     ], [     ] grade to Canadian Lumbermens' Association (CLA) grading rules for Canadian Hardwood Flooring.

.3	Pattern:  [herringbone] [basketweave] [diamond] [swirl] [finger mosaic] [     ].

.4	Sizes of units:  [     ] mm x [     ] mm x [     ] mm thick.

SPEC NOTE:  Standard size of parquet available in CLA grades is 121 x 121 x 8 mm thick.  Verify availability in CLA grades prior to specifying other sizes.

.5	Finish:  [unfinished] [exposed grain "distressed" finish] [prefinished, plastic-impregnated].

.6	Edges:  [tongue and groove] [square] [bevelled].

.7	Moisture Content:  6-9%.

2.3	ACCESSORIES

.1	Adhesive:  to CAN/CGSB-71.27-M88, resilient elastomeric, compatible with substrate and parquet flooring.

SPEC NOTE:  Edit remainder of article to suit project.

.2	Feature [or border] strips:  [     ] mm size, [     ] species.

.3	Threshold:  of same species and grade as wood flooring, [     ] mm wide x full width of opening.

2.4	FINISHING MATERIALS

SPEC NOTE:  Delete if prefinished or plastic impregnated parquet specified.

.1	Finish:  clear [urethane to CAN/CGSB-1.175-M89] [varnish to CAN/CGSB-1.36-M90],  [gloss] [satin] sheen.

.2	Wax:  paste wax to CAN/CGSB-25.2-92.

2.5	FABRICATION

.1	Assemble solid unit blocks of specified wood species and patterns.

.2	Secure individual pieces in blocks together in manner which permits ease of handling and secure installation.

.3	Plastic-impregnated Flooring:  impregnated throughout with methyl methacrylate plastic with colour dye to suit.

SPEC NOTE:  When specifying plastic impregnated wood parquet, delete stain and wax finish.

.4	Sand wearing surface of blocks and finish with [deep penetrating oil stain and] urethane finish.

3.	Execution

3.1	PREPARATION

.1	Examine substrate and verify readiness to receive work of this Section.

.2	Verify moisture content of concrete using the Phenolphthalein Test as recommended by National Oak Flooring Manufacturers Association.

3.2	INSTALLATION, WOOD SUBFLOOR WITH SLEEPERS

SPEC NOTE:  Delete this article when parquet is to be glued directly to concrete or structural wood subfloor.

.1	Apply moisture protection membrane over concrete.  Lap joints minimum 600 mm and seal with purpose-made sheathing tape.  Patch punctures and cuts.

.2	Place sleepers on membrane, parallel to short dimension of room and space at 400 mm nominally, to suit sheathing dimensions.  Stagger sleepers and lap ends 150 mm minimum.

.3	Shim sleepers as required to prevent rocking and to provide solid contact with structural substrate.

.4	Maintain minimum 20 mm expansion gap between sleepers and walls and other vertical surfaces.

.5	Glue and nail plywood sheathing with face grain at right angles to sleepers.

.6	Provide 15 mm expansion gap at plywood subfloor perimeter.

3.3	INSTALLATION, PLYWOOD SUBFLOOR WITHOUT SLEEPERS

SPEC NOTE:  Delete this article when parquet is to be glued directly to concrete or structural wood subfloor.

.1	Apply moisture protection membrane over concrete.  Lap joints minimum 300 mm and seal with mastic or sheathing tape.

.2	Lay plywood sheathing over membranes, stagger end joints 1200 mm.

.3	Leave 10 mm gap at vertical surfaces, except provide 3 mm gap at doors and other locations where wall base will not cover gap.  Provide 5 to 10 mm gap between plywood sheets.

.4	Mechanically fasten plywood to concrete, using minimum 9 fasteners per 1200 x 2400 sheet.

3.4	ADHESIVE APPLICATION

.1	Apply adhesive in accordance with manufacturer's instructions.

.2	Apply no more adhesive than can be covered with flooring before working time expires.

.3	Allow no streaks or lumps in adhesive during application.

.4	Remove adhesive that has set or that has formed surface film before flooring is embedded.  Apply fresh coat of adhesive.

3.5	WOOD PARQUET FLOORING

SPEC NOTE:  Use the first three clauses for rectangular floor areas, or modify as applicable.

.1	Locate first working line approximately parallel to wall exactly 4 or 5 parquet units from flooring edge at entry doorway.

.2	Locate perpendicular centre axis so no unit will be less than one-half size.

.3	Install flooring using pyramid or stair-step sequence starting from intersection of perpendicular axes.

.4	Install flooring units to provide parquet pattern indicated on drawings.  Install flooring units with aligned joints 0.4 mm wide between units.

SPEC NOTE:  Ensure drawings indicate pattern required.

.5	Allow 12 mm expansion space at walls or projections in floor area.

.6	Apply flooring units with uniform pressure.  Avoid working adhesive into joints.

.7	Thoroughly roll metal-splined units until adhesive sets, to ensure full bed adhesion.

.8	At start of installation, remove minimum [6] [  ] units before adhesive sets to inspect for complete coverage of adhesive and replace units immediately.  Adjust installation as required to achieve full bed adhesion.

.9	Remove adhesive visible after installation.

3.6	THRESHOLDS

.1	Install thresholds at openings and where indicated.

.2	Attach threshold to adjacent rigid floor surface.

.3	Threshold to act as ramp between floor surfaces over expansion space.

3.7	SANDING AND FINISHING

SPEC NOTE:  Delete this article when prefinished solid unit blocks are specified.

.1	Power sand floor surface smooth with starting with coarse, progressing to medium and finishing with fine grit.

.2	Vacuum clean.

.3	Apply [two] [three] coats of clear floor finish.

.4	Apply wax to manufacturer's recommendations.

END OF SECTION