Section Cover Page

Section 09699 2007-04-18 Carpet Cleaning

Use this section to specify requirements for limited carpet cleaning in high traffic areas and complete carpet cleaning throughout all floor areas, both as scheduled in another specification section.

This Master Specification Section contains:

.1	This Cover Sheet

.2	Specification Section Text:

1.	General

1.1	Related Requirements

1.2	Performance Requirements

1.3	Maintenance Data

1.4	Testing

1.5	Manufacturer's Instructions

1.6	Cleaner's Qualifications

1.7	Field Test Of Cleaning Method

2.	Products

2.1	Materials

3.	Execution

3.1	Spin Bonnet Method:  High Traffic Area Cleaning

3.2	Steam Clean/Extraction Method:  All Carpet Areas

3.3	Clean-Up

Editing:  Use this Section as is; do not modify in any way.

1.	General

1.1	RELATED REQUIREMENTS

.1	Refer to Caretaking Task and Frequency Schedule for:

.1	which tasks are to be performed, and

.2	frequency at which those tasks are to be performed.

.2	Refer to Carpet Cleaning Materials, Equipment and Location Schedule for:

.1	where operations specified in this Section and in the Task and Frequency Schedule are to be performed,

.2	any materials and equipment to be supplied by the Minister, and

.3	hours of work during which the specified tasks shall be performed.

1.2	PERFORMANCE REQUIREMENTS

.1	Dry vacuuming of carpet areas shall remove all grit, dust, loose and caked dirt.

.2	Spot removing of carpet areas shall remove all isolated stains providing a uniform texture, colour and pattern throughout carpet area.

.3	Spin bonnet method of cleaning for high traffic areas shall remove stains and soiled appearance providing a uniform texture, colour and pattern throughout carpet area.

.4	Steam clean/extraction method of cleaning all carpet shall remove all spots, stains, embedded and surface soil from all carpet areas providing a uniform texture, colour and pattern throughout carpet area.

.5	Wet vacuuming of carpet areas shall remove all excess water, shampoo residues and carpet lint.

1.3	MAINTENANCE DATA

.1	Obtain applicable maintenance data in the Minister's possession for carpet materials scheduled to be cleaned.

1.4	TESTING

.1	Examine carpet and under pad.

.2	Determine material and method of installation.

.3	Provide Minister with a report of recommended method of cleaning that will optimize carpet service life.

.4	Report any areas deteriorated beyond repair, worn, defective, thread bare areas and joints, and an assessment of carpet under pad.

1.5	MANUFACTURER'S INSTRUCTIONS

.1	When available, follow carpet manufacturer's written instructions in the use of cleaning materials and methods for carpet type, material and installation.

1.6	CLEANERS QUALIFICATIONS

.1	Carpet cleaning operations shall be by skilled machine operators appropriate to the cleaning method being employed.

1.7	FIELD TEST OF CLEANING METHOD

.1	Prior to cleaning of complete carpet area or designated high traffic area, clean 1 m2 for Minister's inspection.

.2	Use proposed method of cleaning being used for entire area, in the test area; including vacuuming procedures, detergents, spot removers and cleaning machine operation.

.3	Do not proceed with cleaning of entire area(s) until Minister has inspected test area.

2.	Products

2.1	MATERIALS

.1	Cleaning equipment and supplies being used in carpet cleaning operations shall not be detrimental to carpet fibres, backings and under pads.  Bleaching or bleeding of colours is not acceptable.

3.	Execution

3.1	Spin Bonnet METHOD:  HIGH TRAFFIC AREA CLEANING

.1	Refer to schedule for areas to be cleaned by this method.

.2	Prior to pre-spray application and spin bonnet, vacuum designated high traffic area.

.3	Remove isolated spots and stains.

.4	Apply carpet cleaning solution.  Scrub carpet without creating pile distortion, over wetting of carpet or under pad.

.5	Wet vacuum carpet in 2 directions:

.1	against the grain of normal pile direction, and

.2	in direction of normal pile grain.

.6	Reduce pile distortion to a minimum.

.7	Remove splash marks and detergent residues from bases.

.8	Provide warning signs to wet/damp carpet areas until carpet is dry.

3.2	STEAM CLEAN/EXTRACTION METHOD:  ALL CARPET AREAS

.1	Refer to schedule for areas to be cleaned by this method.

.2	Relocate furniture, equipment, fixtures and storage cartons to an area scheduled by Minister.

.3	Dry vacuum entire floor area to be cleaned.

.4	Remove isolated spots and stains.

.5	Spray steam and apply cleaning shampoo and detergents to carpet without creating any pile distortion, over wetting of carpet or under pad.

.6	In addition to extraction operation provided with application machine, wet vacuum carpet in 2 directions:

.1	against the grain of normal pile direction, and

.2	in direction of normal pile grain.

.7	Reduce pile distortion to a minimum.

.8	Remove splash marks and detergents residue from bases.

.9	Return furniture, equipment, fixtures and storage cartons to their designated locations providing carpet protectors to metal feet and levelers on furniture.  Do not place cardboard cartons on damp or wet carpet.

.10	Provide warning signs to wet/damp carpet areas until carpet is dry.

3.3	CLEAN-UP

.1	Remove all cleaning materials, equipment and supplies from cleaned areas.

END OF SECTION