Section Cover Page

Section 09904 Interior Painting and 2001-08-28 Finishing Schedule

Use this Section to schedule finish systems for all new, interior substrates requiring site painting and finishing, except mechanical and electrical work.

The Alberta Infrastructure Basic Master Specification contains the following series of Sections that specify site painting and finishing:

.1	Section 09901 - Painting and Finishing General Requirements

.2	Section 09902 - Exterior Painting and Finishing Schedule

.3	Section 09903 - Exterior Repainting and Refinishing Schedule

.4	Section 09904 - Interior Painting and Finishing Schedule (this Section)

.5	Section 09905 - Interior Repainting and Refinishing Schedule

.6	Section 09906 - Painting of Mechanical and Electrical Work

These sections are interrelated.  Section 09901 specifies general requirements while the remaining Sections in the series specify finish systems, i.e. specific products and number of coats, required for various types of substrates.  Always include Section 09901.  Include one or more of the remaining Sections only to the extent they are applicable to the project.

This Master Specification Section contains:

.1	This Cover Sheet

.2	Data Sheet - General

.3	Specification Section Text:

1.	General

1.1	Intent

2.	Products

(Not used)

3.	Execution

3.1	Interior Painting and Finishing Schedule

This Section includes a comprehensive listing of various new, interior, substrates complete with MPI code numbers, finishing system descriptions and product descriptions.

Individual finishing systems listed in this master schedule must be edited to suit the project.  Note that the schedule generally calls for 3 coats (premium grade), which is the normal standard for Alberta Infrastructure projects. If 2 coats (custom grade) are deemed to be adequate for a particular project, the schedule needs to be edited accordingly.

Specifier has the option of scheduling painting and finishing by one of two methods:

.1	Method #1:  Indicate in Room Finish Schedule, Door and Frame Schedule, and elsewhere as required, the proper code designation and where applicable, gloss level, of the required finish system for each item or substrate that requires site painting or finishing.  In this Section, specify the finish system corresponding to each required finish.  Coordinate with Room Finish Schedule, Door and Frame Schedule, etc.

.2	Method #2:  Incorporate a comprehensive listing of each item or substrate that requires site finishing into this Section.  Do not indicate code designations elsewhere.  Since this Section describes substrate elements in general terms only, mainly for reference purposes, it may require modification and expansion to include more specific and detailed substrate descriptions.

Carefully consider all surfaces to be painted or finished and ensure that appropriate finishing systems are specified for each surface type. Review the system evaluation and surface preparation section of the MPI Manual when selecting finishing systems.

Specify only those finishing systems that are applicable to the project. Delete all other finishing systems.

END OF DATA SHEET

1.	General

1.1	INTENT

.1	Read this Section in conjunction with Section 09901 - Painting and Finishing General Requirements.

2.	Products

.1	(Not used).

3.	Execution

  - INTERIOR PAINTING AND FINISHING SCHEDULE

.1	The following code numbers, finishing system descriptions, gloss levels, coats and product descriptions are derived from the MPI Architectural Painting Specification Manual and the MPI Approved Products List.

SPEC NOTE: Delete paint systems as required to include only paint systems required for the project. Edit all option fields.  Where more than Gloss Level is required for a particular substrate, include all applicable gloss levels and ensure location of various gloss levels are clearly defined in Section 09901, Article 2.4, Paragraph 2.4.2

- INT 2.1 – ASPHALT SURFACES

| INT 2.1A | Latex Zone / Traffic Marking | Latex Zone/Traffic Marking Paint, Apply in strict accordance with manufacturer’s instructions. |
| --- | --- | --- |
| INT 2.1B | Alkyd Zone / Traffic Marking | Alkyd Zone/Traffic Marking Paint, Apply in strict accordance with manufacturer’s instructions. |

INT 3.1 – CONCRETE VERTICAL SURFACES

| INT 3.1A | Latex, | 1st | Latex Primer Sealer |
| --- | --- | --- | --- |
|  | (over sealer) | 2nd | Latex |
|  | Gloss level [G1] [G2] [G3] [G4] [G5] [G6] | 3rd | Latex |
| INT 3.1B | Latex Aggregate / Latex, | 1st | Latex Aggregate |
|  | [Coarse] [Medium] texture, | 2nd | Latex |
|  | Gloss level [G1] [G5] [G6] | 3rd | Latex |
| INT 3.1C | High Performance Architectural Latex | 1st | Latex Primer Sealer |
|  |  | 2nd | HIPAC Latex |
|  | Gloss level [G2] [G3] [G4] [G5] | 3rd | HIPAC Latex |
| INT 3.1D | Alkyd | 1st | Latex Primer Sealer |
|  | Gloss level [G1] [G3] [G5] [G6] | 2nd | Alkyd |
|  |  | 3rd | Alkyd |
| INT 3.1E | Latex | 1st | Latex |
|  | Gloss level [G1] [G2] [G3] [G4] [G5] [G6] | 2nd | Latex |
|  |  | 3rd | Latex |
| INT 3.1F | Epoxy | 1st | Epoxy |
|  | (for smooth concrete) | 2nd | Epoxy |
|  |  | 3rd | Epoxy |
| INT 3.1G | Epoxy, Water Based | 1st | W. B. Epoxy |
|  | (for smooth concrete) | 2nd | W. B. Epoxy |
|  |  | 3rd | W. B. Epoxy |
| INT 3.1H | Multicolor | 1st | Primer Sealer |
|  |  | 2nd | Multicolor Base coat |
|  |  | 3rd | Multicolor Pattern Coat |
|  |  | 4th | Clear Coat |
| INT 3.1J | Water Repellent |  | Water Repellent, Apply in strict accordance with manufacturer’s instructions. |
|  | (paintable) |  |  |
| INT 3.1K | Concrete Stain |  | Concrete Stain, Apply in strict accordance with manufacturer’s instructions. |
| INT 3.1L | Waterborne Light Industrial Coating | 1st | W. B. Light Industrial Coating |
|  | Gloss level [G3] [G5] [G6] | 2nd | W. B. Light Industrial Coating |
|  |  | 3rd | W. B. Light Industrial Coating |
| INT 3.1M | Institutional Low Odor / VOC | 1st | Institutional Low Odor / VOC |
|  | Gloss level [G1] [G2] [G3] [G4] [G5] [G6] | 2nd | Institutional Low Odor / VOC |
|  |  | 3rd | Institutional Low Odor / VOC |
| INT 3.1N | Latex Aggregate, |  | Latex Aggregate, Apply in strict accordance with manufacturer’s instructions. |
|  | [Coarse] [Medium] texture, |  |  |
|  | Gloss level [G1] [G5] [G6] |  |  |

INT 3.2 – CONCRETE HORIZONTAL SURFACES

| INT 3.2A | Latex Floor Enamel | 1st | Latex Floor |
| --- | --- | --- | --- |
|  | Gloss level [G3] [G6] | 2nd | Latex Floor |
|  |  | 3rd | Latex Floor |
| INT 3.2B | Alkyd Floor Paint | 1st | Alkyd Floor |
|  | Gloss level [G4] [G6] | 2nd | Alkyd Floor |
|  |  | 3rd | Alkyd Floor |
| INT 3.2C | Epoxy | 1st | Epoxy |
|  |  | 2nd | Epoxy |
|  |  | 3rd | Epoxy |
| INT 3.2D | Polyurethane, Pigmented | 1st | Epoxy |
|  |  | 2nd | Polyurethane |
|  |  | 3rd | Polyurethane |
| INT 3.2E | Concrete Stain |  | Concrete Stain, Apply in strict accordance with manufacturer’s instructions. |
| INT 3.2F | Concrete Floor Sealer | 1st | Concrete Floor Sealer |
|  |  | 2nd | Concrete Floor Sealer |
| INT 3.2G | Concrete Floor Sealer, Water Based | 1st | Concrete Floor Sealer |
|  |  | 2nd | Concrete Floor Sealer |
| INT 3.2H | Latex Zone / Traffic Marking |  | Latex, Apply in strict accordance with manufacturer’s instructions. |
| INT 3.2J | Alkyd Zone / Traffic Marking |  | Alkyd, Apply in strict accordance with manufacturer’s instructions. |
| INT 3.2K | Polyurethane, Clear, 2 Component | 1st | Polyurethane |
|  |  | 2nd | Polyurethane |
|  |  | 3rd | Polyurethane |
| INT 3.2L | Epoxy Floor Paint, Water Based | 1st | W. B. Epoxy Floor Paint |
|  |  | 2nd | W. B. Epoxy Floor Paint |
|  |  | 3rd | W. B. Epoxy Floor Paint |

INT 3.3 – CEMENTITIOUS COMPOSITION BOARD

| INT 3.3A | Latex | 1st | Latex |
| --- | --- | --- | --- |
|  | Gloss level [G1] [G2] [G3] [G4] [G5] [G6] | 2nd | Latex |
|  |  | 3rd | Latex |
| INT 3.3B | High Performance Architectural Latex | 1st | HIPAC Latex |
|  |  | 2nd | HIPAC Latex |
|  | Gloss level [G2] [G3] [G4] [G5] | 3rd | HIPAC Latex |
| INT 3.3C | Alkyd | 1st | Latex Primer Sealer |
|  | Gloss level [G1] [G3] [G5] [G6] | 2nd | Alkyd |
|  |  | 3rd | Alkyd |
| INT 3.3D | Epoxy, Water Based | 1st | W. B. Epoxy |
|  |  | 2nd | W. B. Epoxy |
|  |  | 3rd | W. B. Epoxy |
| INT 3.3E | Epoxy | 1st | Epoxy |
|  |  | 2nd | Epoxy |
|  |  | 3rd | Epoxy |
| INT 3.3F | Multicolor | 1st | Primer Sealer |
|  |  | 2nd | Multicolor Base coat |
|  |  | 3rd | Multicolor Pattern Coat |
|  |  | 4th | Clear Coat |
| INT 3.3G | Institutional Low Odor / VOC | 1st | Institutional Low Odor / VOC |
|  | Gloss level [G1] [G2] [G3] [G4] [G5] [G6] | 2nd | Institutional Low Odor / VOC |
|  |  | 3rd | Institutional Low Odor / VOC |
| INT 3.3H | Waterborne Light Industrial Coating | 1st | W. B. Light Industrial Coating |
|  | Gloss level [G3] [G5] [G6] | 2nd | W. B. Light Industrial Coating |
|  |  | 3rd | W. B. Light Industrial Coating |

INT 4.1 – CLAY MASONRY UNITS

| INT 4.1A | Latex | 1st | Latex |
| --- | --- | --- | --- |
|  | Gloss level [G1] [G2] [G3] [G4] [G5] [G6] | 2nd | Latex |
|  |  | 3rd | Latex |
| INT 4.1B | Latex Aggregate |  | Latex Aggregate, Apply in strict accordance with manufacturer’s instructions. |
|  | [Coarse] [Medium] texture, |  |  |
|  | Gloss level [G1] [G5] [G6] |  |  |
| INT 4.1C | Waterborne Light Industrial Coating | 1st | W. B. Light Industrial Coating |
|  | Gloss level [G3] [G5] [G6] | 2nd | W. B. Light Industrial Coating |
|  |  | 3rd | W. B. Light Industrial Coating |
| INT 4.1D | Alkyd | 1st | Latex Primer Sealer |
|  | Gloss level [G1] [G3] [G5] [G6] | 2nd | Alkyd |
|  |  | 3rd | Alkyd |
| INT 4.1F | Epoxy | 1st | Epoxy |
|  | (for smooth brick) | 2nd | Epoxy |
|  |  | 3rd | Epoxy |
| INT 4.1G | Epoxy, Water Based | 1st | W. B. Epoxy |
|  | (for smooth brick) | 2nd | W. B. Epoxy |
|  |  | 3rd | W. B. Epoxy |
| INT 4.1H | Multicolor | 1st | Primer Sealer |
|  |  | 2nd | Multicolor Base coat |
|  |  | 3rd | Multicolor Pattern Coat |
|  |  | 4th | Clear Coat |
| INT 4.1J | Water Repellent, Clear |  | Water Repellent, Apply in strict accordance with manufacturer’s instructions |
|  | (paintable) |  |  |
| INT 4.1K | Polyurethane, Clear, 2 Component | 1st | Polyurethane |
|  |  | 2nd | Polyurethane |
|  |  | 3rd | Polyurethane |
| INT 4.1L | High Performance Architectural Latex | 1st | HIPAC Latex |
|  |  | 2nd | HIPAC Latex |
|  | Gloss level [G2] [G3] [G4] [G5] | 3rd | HIPAC Latex |
| INT 4.1M | Institutional Low Odor / VOC | 1st | Institutional Low Odor / VOC |
|  | Gloss level [G1] [G2] [G3] [G4] [G5] [G6] | 2nd | Institutional Low Odor / VOC |
|  |  | 3rd | Institutional Low Odor / VOC |

INT 4.2 – CONCRETE MASONRY UNITS (block and brick)

| INT 4.2A | Latex | 1st | Block Filler |
| --- | --- | --- | --- |
|  | Gloss level [G1] [G2] [G3] [G4] [G5] [G6] | 2nd | Latex |
|  |  | 3rd | Latex |
| INT 4.2B | Latex Aggregate |  | Latex Aggregate, Apply in strict accordance with manufacturer’s instructions. |
|  | [Coarse] [Medium] texture, |  |  |
|  | Gloss level [G1] [G5] [G6] |  |  |
| INT 4.2C | Alkyd | 1st | Latex Block Filler |
|  | Gloss level [G1] [G3] [G5] [G6] | 2nd | Alkyd |
|  |  | 3rd | Alkyd |
| INT 4.2D | High Performance Architectural Latex | 1st | Latex Block Filler |
|  |  | 2nd | HIPAC |
|  | Gloss level [G2] [G3] [G4] [G5] | 3rd | HIPAC |
| INT 4.2E | Institutional Low Odor / VOC | 1st | Latex Block Filler |
|  | Gloss level [G1] [G2] [G3] [G4] [G5] [G6] | 2nd | Institutional Low Odor / VOC |
|  |  | 3rd | Institutional Low Odor / VOC |
| INT 4.2F | Epoxy | 1st | Latex Block Filler |
|  | (for dry environments) | 2nd | Epoxy |
|  |  | 3rd | Epoxy |
| INT 4.2G | Epoxy | 1st | Epoxy Block Filler |
|  | (for wet environments) | 2nd | Epoxy |
|  |  | 3rd | Epoxy |
| INT 4.2H | Multicolor | 1st | Latex Block Filler |
|  |  | 2nd | Primer Sealer |
|  |  | 3rd | Multicolor Base coat |
|  |  | 4th | Multicolor Pattern Coat |
|  |  | 5th | Clear Coat |
| INT 4.2J | Epoxy, Water Based | 1st | Latex Block Filler |
|  | (for dry environments) | 2nd | W. B. Epoxy |
|  |  | 3rd | W. B. Epoxy |
| INT 4.2K | Waterborne Light Industrial Coating | 1st | Latex Block Filler |
|  | Gloss level [G3] [G5] [G6] | 2nd | W. B. Light Industrial Coating |
|  |  | 3rd | W. B. Light Industrial Coating |
| INT 4.2L | Water Repellent |  | Water Repellent, Apply in strict accordance with manufacturer’s instructions. |
|  | (not paintable) |  |  |
|  | (do not use on low density block) |  |  |
| INT 4.2M | Water Repellent |  | Water Repellent, Apply in strict accordance with manufacturer’s instructions. |
|  | (paintable) |  |  |
|  | (do not use on low density block) |  |  |
| INT 4.2N | Alkyd | 1st | Latex Block Filler |
|  | (over latex sealer) | 2nd | Latex Primer Sealer |
|  | Gloss level [G1] [G3] [G5] [G6] | 3rd | Alkyd |
|  |  | 4th | Alkyd |

INT 5.1 – STRUCTURAL STEEL & METAL FABRICATIONS

| INT 5.1A | Quick Dry Enamel | 1st | Quick Dry Metal Primer |
| --- | --- | --- | --- |
|  | Gloss level [G5] [G6] | 2nd | Quick Dry Enamel |
|  |  | 3rd | Quick Dry Enamel |
| INT 5.1B | Waterborne Light Industrial Coating | 1st | Rust Inhibitive Primer |
|  | Gloss Level [G3] [G5] [G6] | 2nd | W.B. Light Industrial Coating |
|  |  | 3rd | W.B. Light Industrial Coating |
| INT 5.1C | Waterborne Dry Fall | 1st | Alkyd Metal Primer |
|  |  | 2nd | Waterborne Dry Fall |
| INT 5.1D | Alkyd Dry Fall | 1st | Alkyd Metal Primer |
|  |  | 2nd | Alkyd Dry Fall |
| INT 5.1E | Alkyd | 1st | Alkyd Metal Primer |
|  | Gloss level [G1] [G3] [G5] [G6] | 2nd | Alkyd |
|  |  | 3rd | Alkyd |
| INT 5.1F | Polyurethane, Pigmented | 1st | Epoxy Primer |
|  | (over epoxy primer) | 2nd | Polyurethane |
|  |  | 3rd | Polyurethane |
| INT 5.1G | Polyurethane, Pigmented | 1st | Epoxy Primer |
|  | (over high build epoxy) | 2nd | High Build Epoxy |
|  |  | 3rd | Polyurethane |
| INT 5.1H | Polyurethane, Pigmented | 1st | Inorganic Zinc Primer |
|  | (over inorganic zinc and epoxy) | 2nd | Epoxy |
|  |  | 3rd | Polyurethane |
| INT 5.1J | Polyurethane, Pigmented | 1st | Epoxy Zinc Rich Primer |
|  | (over epoxy zinc rich and epoxy) | 2nd | Epoxy |
|  |  | 3rd | Polyurethane |
| INT 5.1K | Epoxy, Water Based | 1st | Rust Inhibitive Primer |
|  |  | 2nd | W. B. Epoxy |
|  |  | 3rd | W. B. Epoxy |
| INT 5.1L | Epoxy | 1st | Epoxy Primer |
|  |  | 2nd | Epoxy |
|  |  | 3rd | Epoxy |
| INT 5.1M | Aluminum Paint | 1st | Alkyd Metal Primer |
|  |  | 2nd | Aluminum Paint |
|  |  | 3rd | Aluminum Paint |
| INT 5.1N | Waterborne Light Industrial Coating | 1st | Epoxy Primer |
|  | (over epoxy primer) | 2nd | W.B. Light Industrial Coating |
|  | Gloss Level [G3] [G5] [G6] | 3rd | W.B. Light Industrial Coating |
| INT 5.1P | High Build Epoxy | 1st | Epoxy Zinc Rich Primer |
|  | (over epoxy zinc rich primer) | 2nd | High Build Epoxy |
|  |  | 3rd | Epoxy |
| INT 5.1Q | Latex, (over alkyd primer) | 1st | Alkyd Metal Primer |
|  | Gloss level [G1] [G2] [G3] [G4] [G5] [G6] | 2nd | Latex |
|  |  | 3rd | Latex |
| INT 5.1R | High Performance Architectural Latex | 1st | Alkyd Metal Primer |
|  |  | 2nd | HIPAC Latex |
|  | Gloss level [G2] [G3] [G4] [G5] | 3rd | HIPAC Latex |
| INT 5.1S | Institutional Low Odor / VOC | 1st | Rust Inhibitive Primer |
|  | Gloss level [G1] [G2] [G3] [G4] [G5] [G6] | 2nd | Institutional Low Odor / VOC |
|  |  | 3rd | Institutional Low Odor / VOC |

INT 5.2 – STEEL – HIGH HEAT

| INT 5.2A | Heat Resistant Enamel | Heat Resistant Enamel, Apply in strict accordance with Manufacturer’s Instructions |
| --- | --- | --- |
|  | Maximum 205°C (400°F) |  |
| INT 5.2B | Heat Resistant Enamel, Aluminum | Aluminum Heat Resistant Enamel, Apply in strict accordance with Manufacturer’s Instructions |
|  | Maximum 427°C (800°F) |  |
| INT 5.2C | Inorganic Zinc Rich Coating | Inorganic Zinc Rich Coating, Apply in strict accordance with Manufacturer’s Instructions |
|  | Maximum 400°C (750°F) |  |
| INT 5.2D | High Heat Resistant Coating | High Heat Resistant Coating, Apply in strict accordance with Manufacturer’s Instructions |
|  | Maximum 593°C (1100°F) |  |

INT 5.3 – GALVANIZED METAL (not chromate passified)

| INT 5.3A | Latex | 1st | Cementitious Primer |
| --- | --- | --- | --- |
|  | Gloss Level [G1] [G2] [G3] [G4] [G5] [G6] | 2nd | Latex |
|  |  | 3rd | Latex |
| INT 5.3B | Waterborne Light Industrial Coating | 1st | Cementitious Primer |
|  | Gloss Level [G3] [G5] [G6] | 2nd | W.B. Light Industrial Coating |
|  |  | 3rd | W.B. Light Industrial Coating |
| INT 5.3C | Alkyd | 1st | Cementitious Primer |
|  | Gloss level [G1] [G3] [G5] [G6] | 2nd | Alkyd |
|  |  | 3rd | Alkyd |
| INT 5.3D | Epoxy | 1st | Epoxy Primer |
|  | (over epoxy primer) | 2nd | Epoxy |
|  |  | 3rd | Epoxy |
| INT 5.3E | Epoxy | 1st | Vinyl Wash Primer |
|  | (over vinyl wash and epoxy primer) | 2nd | Epoxy Primer |
|  |  | 3rd | Epoxy |
|  |  | 4th | Epoxy |
| INT 5.3F | Alkyd Dry Fall | 1st | Cementitious Primer |
|  | (low contact / traffic) | 2nd | Alkyd Dry Fall |
| INT 5.3G | Aluminum Paint | 1st | Cementitious Primer |
|  |  | 2nd | Aluminum Paint |
|  |  | 3rd | Aluminum Paint |
| INT 5.3H | Waterborne Dry Fall | 1st | Alkyd Metal Primer |
|  | (low contact / traffic) | 2nd | Waterborne Dry Fall |
| INT 5.3J | Latex | 1st | W. B. Primer |
|  | (over w.b. primer) | 2nd | Latex |
|  | Gloss Level [G1] [G2] [G3] [G4] [G5] [G6] | 3rd | Latex |
| INT 5.3K | Waterborne Light Industrial Coating | 1st | Waterborne Primer |
|  | (over w. b. primer) | 2nd | W.B. Light Industrial Coating |
|  | Gloss Level [G3] [G5] [G6] | 3rd | W.B. Light Industrial Coating |
| INT 5.3L | Alkyd | 1st | Non-cementitious Primer |
|  | (over non-cementitious primer) | 2nd | Alkyd |
|  | Gloss level [G1] [G3] [G5] [G6] | 3rd | Alkyd |
| INT 5.1M | High Performance Architectural Latex | 1st | Waterborne Galvanized Primer |
|  |  | 2nd | HIPAC Latex |
|  | Gloss level [G2] [G3] [G4] [G5] | 3rd | HIPAC Latex |
| INT 5.1N | Institutional Low Odor / VOC | 1st | Waterborne Galvanized Primer |
|  | Gloss level [G1] [G2] [G3] [G4] [G5] [G6] | 2nd | Institutional Low Odor / VOC |
|  |  | 3rd | Institutional Low Odor / VOC |

INT 5.4 – ALUMINUM (non-anodized)

| INT 5.4A | Alkyd | 1st | Vinyl Wash Primer |
| --- | --- | --- | --- |
|  | Gloss Level [G1] [G3] [G5] [G6] | 2nd | Alkyd |
|  |  | 3rd | Alkyd |
| INT 5.4B | Epoxy | 1st | Vinyl Wash Primer |
|  |  | 2nd | Epoxy |
|  |  | 3rd | Epoxy |
| INT 5.4C | Polyurethane, Pigmented | 1st | Vinyl Wash Primer |
|  | (over epoxy) | 2nd | Epoxy |
|  |  | 3rd | Polyurethane |
| INT 5.4D | Aluminum Paint | 1st | Vinyl Wash Primer |
|  |  | 2nd | Aluminum Paint |
|  |  | 3rd | Aluminum Paint |
| INT 5.4E | Waterborne Light Industrial Coating | 1st | Quick Dry Primer |
|  | Gloss Level [G3] [G5] [G6] | 2nd | W.B. Light Industrial Coating |
|  |  | 3rd | W.B. Light Industrial Coating |
| INT 5.4F | High Performance Architectural Latex | 1st | Quick Dry Primer |
|  |  | 2nd | HIPAC Latex |
|  | Gloss level [G2] [G3] [G4] [G5] | 3rd | HIPAC Latex |
| INT 5.4G | Institutional Low Odor / VOC | 1st | Quick Dry Primer |
|  | Gloss level [G1] [G2] [G3] [G4] [G5] [G6] | 2nd | Institutional Low Odor / VOC |
|  |  | 3rd | Institutional Low Odor / VOC |
| INT 5.4H | Latex | 1st | Quick Dry Primer |
|  | Gloss Level [G1] [G2] [G3] [G4] [G5] [G6] | 2nd | Latex |
|  |  | 3rd | Latex |

INT 5.5 – COPPER

| INT 5.5A | Alkyd | 1st | Vinyl Wash Primer |
| --- | --- | --- | --- |
|  | Gloss Level [G1] [G3] [G5] [G6] | 2nd | Alkyd |
|  |  | 3rd | Alkyd |
| INT 5.5B | Epoxy | 1st | Vinyl Wash Primer |
|  |  | 2nd | Epoxy |
|  |  | 3rd | Epoxy |
| INT 5.5C | Polyurethane, Pigmented | 1st | Vinyl Wash Primer |
|  | (over epoxy) | 2nd | Epoxy |
|  |  | 3rd | Polyurethane |
| INT 5.5D | Aluminum Paint | 1st | Vinyl Wash Primer |
|  |  | 2nd | Aluminum Paint |
|  |  | 3rd | Aluminum Paint |
| INT 5.5E | Waterborne Light Industrial Coating | 1st | Quick Dry Primer |
|  | Gloss Level [G3] [G5] [G6] | 2nd | W.B. Light Industrial Coating |
|  |  | 3rd | W.B. Light Industrial Coating |
| INT 5.5F | High Performance Architectural Latex | 1st | Quick Dry Primer |
|  |  | 2nd | HIPAC Latex |
|  | Gloss level [G2] [G3] [G4] [G5] | 3rd | HIPAC Latex |
| INT 5.5G | Institutional Low Odor / VOC | 1st | Quick Dry Primer |
|  | Gloss level [G1] [G2] [G3] [G4] [G5] [G6] | 2nd | Institutional Low Odor / VOC |
|  |  | 3rd | Institutional Low Odor / VOC |
| INT 5.5H | Latex | 1st | Quick Dry Primer |
|  | Gloss level [G1] [G2] [G3] [G4] [G5] [G6] | 2nd | Latex |
|  |  | 3rd | Latex |

INT 5.6 – STAINLESS STEEL (not polished)

| INT 5.6A | Waterborne Light Industrial Coating | 1st | Bonding Primer |
| --- | --- | --- | --- |
|  | (over bonding primer) | 2nd | W.B. Light Industrial Coating |
|  | Gloss Level [G3] [G5] [G6] | 3rd | W.B. Light Industrial Coating |
| INT 5.6B | Alkyd | 1st | Vinyl Wash Primer |
|  | Gloss Level [G1] [G3] [G5] [G6] | 2nd | Alkyd |
|  |  | 3rd | Alkyd |
| INT 5.6C | Epoxy | 1st | Vinyl Wash Primer |
|  |  | 2nd | Epoxy |
|  |  | 3rd | Epoxy |
| INT 5.6D | Polyurethane, Pigmented | 1st | Vinyl Wash Primer |
|  |  | 2nd | Epoxy |
|  |  | 3rd | Polyurethane |
| INT 5.6E | Aluminum Paint | 1st | Vinyl Wash Primer |
|  |  | 2nd | Aluminum Paint |
|  |  | 3rd | Aluminum Paint |
| INT 5.6F | Waterborne Light Industrial Coating | 1st | Quick Dry Primer |
|  | (over q. d. primer) | 2nd | W.B. Light Industrial Coating |
|  | Gloss Level [G3] [G5] [G6] | 3rd | W.B. Light Industrial Coating |
| INT 5.6G | High Performance Architectural Latex | 1st | Bonding Primer |
|  |  | 2nd | HIPAC Latex |
|  | Gloss level [G2] [G3] [G4] [G5] | 3rd | HIPAC Latex |
| INT 5.6H | Latex | 1st | Bonding Primer |
|  | Gloss level [G1] [G2] [G3] [G4] [G5] [G6] | 2nd | Latex |
|  |  | 3rd | Latex |

INT 6.1 – GLUE LAMINATED BEAMS AND COLUMNS

| INT 6.1A | Latex | 1st |  | Alkyd Primer Sealer |
| --- | --- | --- | --- | --- |
|  | (over alkyd primer) | 2nd |  | Latex |
|  | Gloss level [G1] [G2] [G3] [G4] [G5] [G6] | 3rd |  | Latex |
| INT 6.1B | Alkyd | 1st |  | Alkyd Primer Sealer |
|  | Gloss Level [G1] [G3] [G5] [G6] | 2nd |  | Alkyd |
|  |  | 3rd |  | Alkyd |
| INT 6.1C | Alkyd Varnish | 1st |  | Alkyd Primer Sealer |
|  | (primer sealer factory applied) | 2nd |  | Alkyd Varnish |
|  | Gloss Level [G1] [G5] [G6] | 3rd |  | Alkyd Varnish |
| INT 6.1D | Polyurethane Varnish | 1st |  | Polyurethane |
|  | (first coat factory applied) | 2nd |  | Polyurethane |
|  | Gloss Level [G4] [G6] | 3rd |  | Polyurethane |
| INT 6.1E | Polyurethane, Pigmented | 1st |  | Polyurethane |
|  |  | 2nd |  | Polyurethane |
|  |  | 3rd |  | Polyurethane |
| INT 6.1F | Waterborne Acrylic, Clear | 1st |  | W. B. Acrylic |
|  | Gloss Level [G4] [G5] [G6] | 2nd |  | W. B. Acrylic |
|  |  | 3rd |  | W. B. Acrylic |
| INT 6.1G | Semi-transparent Stain | 1st |  | Semi-transparent Stain |
|  |  | 2nd |  | Semi-transparent Stain |
| INT 6.1H | Alkyd Solid Color Stain | 1st |  | Solid Color Wood Stain |
|  |  | 2nd |  | Solid Color Wood Stain |
| INT 6.1J | Polyurethane Varnish | 1st |  | Wood Stain |
|  | (over stain) | 2nd |  | Polyurethane Varnish |
|  | Gloss Level [G4] [G6] | 3rd |  | Polyurethane Varnish |
|  |  | 4th |  | Polyurethane Varnish |
| INT 6.1K | Alkyd Varnish | 1st |  | Semi-transparent Stain |
|  | (over stain) | 2nd |  | Alkyd Varnish |
|  | Gloss Level [G1] [G5] [G6] | 3rd |  | Alkyd Varnish |
|  |  | 4th |  | Alkyd Varnish |
| INT 6.1L | Epoxy | 1st |  | Epoxy |
|  |  | 2nd |  | Epoxy |
|  |  | 3rd |  | Epoxy |
| INT 6.1M | Latex | 1st |  | Latex Primer |
|  | (over latex primer) | 2nd |  | Latex |
|  | Gloss level [G1] [G2] [G3] [G4] [G5] [G6] | 3rd |  | Latex |
| INT 6.1N | High Performance Architectural Latex | 1st | Latex Primer |  |
|  |  | 2nd | HIPAC Latex |  |
|  | Gloss level [G2] [G3] [G4] [G5] | 3rd |  | HIPAC Latex |
| INT 6.1P | Alkyd Varnish | 1st |  | Semi-transparent Stain |
|  | (over stain and sealer) | 2nd |  | Sanding Sealer |
|  | Gloss Level [G1] [G5] [G6] | 3rd |  | Alkyd Varnish |
|  |  | 4th |  | Alkyd Varnish |
| INT 6.1Q | Institutional Low Odor / VOC | 1st |  | Latex Primer |
|  | Gloss level [G1] [G2] [G3] [G4] [G5] [G6] | 2nd |  | Institutional Low Odor / VOC |
|  |  | 3rd |  | Institutional Low Odor / VOC |
| INT 6.1R | Waterborne Acrylic, Clear | 1st |  | Semi-transparent Stain |
|  | Gloss Level [G4] [G5] [G6] | 2nd |  | W. B. Acrylic |
|  |  | 3rd |  | W. B. Acrylic |
|  |  | 4th |  | W. B. Acrylic |
| INT 6.1S | Polyurethane, Clear, Moisture Cured | 1st |  | Semi-transparent Stain |
|  | (over stain) | 2nd |  | Polyurethane |
|  | Gloss Level [G1] [G6] | 3rd |  | Polyurethane |
|  |  | 4th |  | Polyurethane |
| INT 6.1T | Latex Solid Color Stain | 1st |  | Oil/Alkyd Primer |
|  |  | 2nd |  | Latex Solid Stain |
|  |  | 3rd |  | Latex Solid Stain |
| INT 6.1U | Fire Retardant, Pigmented |  |  | (ULC approved), Apply in strict accordance with Manufacturer’s Instructions |
| INT 6.1V | Fire Retardant, Clear |  |  | (ULC approved), Apply in strict accordance with Manufacturer’s Instructions |

INT 6.2 – DIMENSION LUMBER

| INT 6.2A | Latex | 1st | Alkyd Primer Sealer |
| --- | --- | --- | --- |
|  | (over alkyd primer) | 2nd | Latex |
|  | Gloss level [G1] [G2] [G3] [G4] [G5] [G6] | 3rd | Latex |
| INT 6.2B | High Performance Architectural Latex | 1st | Alkyd Primer Sealer |
|  |  | 2nd | HIPAC Latex |
|  | Gloss level [G2] [G3] [G4] [G5] | 3rd | HIPAC Latex |
| INT 6.2C | Alkyd | 1st | Alkyd Primer Sealer |
|  | Gloss Level [G1] [G3] [G5] [G6] | 2nd | Alkyd |
|  |  | 3rd | Alkyd |
| INT 6.2D | Latex | 1st | Latex Primer |
|  | (over latex primer) | 2nd | Latex |
|  | Gloss level [G1] [G2] [G3] [G4] [G5] [G6] | 3rd | Latex |
| INT 6.2E | Multicolor | 1st | Alkyd Primer Sealer |
|  |  | 2nd | Multicolor Base coat |
|  |  | 3rd | Multicolor Pattern Coat |
|  |  | 4th | Clear Coat |
| INT 6.2F | Fire Retardant, Pigmented |  | (ULC approved), Apply in strict accordance with Manufacturer’s Instructions |
| INT 6.2G | Fire Retardant, Clear |  | (ULC approved), Apply in strict accordance with Manufacturer’s Instructions |
| INT 6.2H | Polyurethane Varnish | 1st | Polyurethane Varnish |
|  | Gloss Level [G4] [G6] | 2nd | Polyurethane Varnish |
|  |  | 3rd | Polyurethane Varnish |
| INT 6.2J | Polyurethane Varnish | 1st | Wood Stain |
|  | (over stain) | 2nd | Polyurethane Varnish |
|  | Gloss Level [G4] [G6] | 3rd | Polyurethane Varnish |
|  |  | 4th | Polyurethane Varnish |
| INT 6.2K | Alkyd Varnish | 1st | Semi-transparent Stain |
|  | (over stain and sealer) | 2nd | Sanding Sealer |
|  | Gloss Level [G1] [G5] [G6] | 3rd | Alkyd Varnish |
|  |  | 4th | Alkyd Varnish |
| INT 6.2L | Institutional Low Odor / VOC | 1st | Latex Primer |
|  | Gloss level [G1] [G2] [G3] [G4] [G5] [G6] | 2nd | Institutional Low Odor / VOC |
|  |  | 3rd | Institutional Low Odor / VOC |
| INT 6.2M | Waterborne Acrylic, Clear | 1st | Semi-transparent Stain |
|  | (over stain) | 2nd | W. B. Acrylic |
|  | Gloss Level [G4] [G5] [G6] | 3rd | W. B. Acrylic |
|  |  | 4th | W. B. Acrylic |
| INT 6.2N | Polyurethane, Clear, Moisture Cured | 1st | Semi-transparent Stain |
|  | (over stain) | 2nd | Polyurethane |
|  | Gloss Level [G1] [G6] | 3rd | Polyurethane |
|  |  | 4th | Polyurethane |
| INT 6.2P | Alkyd Varnish | 1st | Sanding Sealer |
|  | (primer sealer factory applied) | 2nd | Alkyd Varnish |
|  | Gloss Level [G1] [G5] [G6] | 3rd | Alkyd Varnish |

INT 6.3 – DRESSED LUMBER

| INT 6.3A | High Performance Architectural Latex | 1st | Latex Primer Sealer |
| --- | --- | --- | --- |
|  |  | 2nd | HIPAC Latex |
|  | Gloss level [G2] [G3] [G4] [G5] | 3rd | HIPAC Latex |
| INT 6.3B | Alkyd | 1st | Alkyd Primer Sealer |
|  | Gloss Level [G3] [G5] [G6] | 2nd | Alkyd |
|  |  | 3rd | Alkyd |
| INT 6.3C | Semi-transparent Stain | 1st | Wood Stain |
|  | (not for use on doors) | 2nd | Wood Stain |
| INT 6.3D | Alkyd Varnish | 1st | Wood Stain |
|  | (over stain) | 2nd | Sanding Sealer / Shellac |
|  | Gloss Level [G5] [G6] | 3rd | Alkyd Varnish |
|  |  | 4th | Alkyd Varnish |
| INT 6.3E | Polyurethane Varnish | 1st | Wood Stain |
|  | (over stain) | 2nd | Polyurethane Varnish |
|  | Gloss Level [G4] [G6] | 3rd | Polyurethane Varnish |
|  |  | 4th | Polyurethane Varnish |
| INT 6.3F | Lacquer, Clear | 1st | Wood Stain |
|  | (over stain) | 2nd | Lacquer Sanding Sealer |
|  | Gloss Level [G1] [G4] [G6] | 3rd | Clear Lacquer |
|  |  | 4th | Clear Lacquer |
| INT 6.3G | Lacquer, Pigmented | 1st | Lacquer Sanding Sealer |
|  | Gloss Level [G4] [G5] [G6] | 2nd | Pigmented Lacquer |
|  |  | 3rd | Pigmented Lacquer |
|  |  | 4th | Pigmented Lacquer |
| INT 6.3H | Lacquer, Clear | 1st | Lacquer Sanding Sealer |
|  | Gloss Level [G1] [G4] [G6] | 2nd | Clear Lacquer |
|  |  | 3rd | Clear Lacquer |
|  |  | 4th | Clear Lacquer |
| INT 6.3J | Alkyd Varnish | 1st | Sanding Sealer / Shellac |
|  | Gloss Level [G5] [G6] | 2nd | Alkyd Varnish |
|  |  | 3rd | Alkyd Varnish |
| INT 6.3K | Polyurethane Varnish | 1st | Polyurethane Varnish |
|  | Gloss Level [G4] [G6] | 2nd | Polyurethane Varnish |
|  |  | 3rd | Polyurethane Varnish |
| INT 6.3L | Epoxy | 1st | Epoxy |
|  |  | 2nd | Epoxy |
|  |  | 3rd | Epoxy |
| INT 6.3M | Danish Oil | 1st | Danish Oil |
|  |  | 2nd | Danish Oil |
| INT 6.3N | Multicolor | 1st | Alkyd Primer Sealer (tinted) |
|  |  | 2nd | Multicolor Base coat |
|  |  | 3rd | Multicolor Pattern Coat |
|  |  | 4th | Clear Coat |
| INT 6.3P | Waterborne Light Industrial Coating | 1st | Alkyd Primer Sealer |
|  | Gloss Level [G5] [G6] | 2nd | W. B. Light Industrial Coating |
|  |  | 3rd | W. B. Light Industrial Coating |
| INT 6.3Q | Waterborne Acrylic, Clear | 1st | W. B. Acrylic |
|  | Gloss Level [G4] [G5] [G6] | 2nd | W. B. Acrylic |
|  |  | 3rd | W. B. Acrylic |
| INT 6.3R | Fire Retardant, Pigmented |  | (ULC approved), Apply in strict accordance with Manufacturer’s Instructions |
| INT 6.3S | Fire Retardant, Clear |  | (ULC approved), Apply in strict accordance with Manufacturer’s Instructions |
| INT 6.3T | Latex | 1st | Latex Primer |
|  | (over latex primer) | 2nd | Latex |
|  | Gloss Level [G5] [G6] | 3rd | Latex |
| INT 6.3U | Latex | 1st | Alkyd Primer Sealer |
|  | (over alkyd primer) | 2nd | Latex |
|  | Gloss Level [G5] [G6] | 3rd | Latex |
| INT 6.3V | Institutional Low Odor / VOC | 1st | Latex Primer |
|  | Gloss level [G1] [G2] [G3] [G4] [G5] [G6] | 2nd | Institutional Low Odor / VOC |
|  |  | 3rd | Institutional Low Odor / VOC |
| INT 6.3W | Waterborne Acrylic, Clear | 1st | Semi-transparent Stain |
|  | (over stain) | 2nd | W. B. Acrylic |
|  | Gloss Level [G4] [G5] [G6] | 3rd | W. B. Acrylic |
|  |  | 4th | W. B. Acrylic |
| INT 6.3X | Polyurethane, Clear, Moisture Cured | 1st | Polyurethane |
|  | Gloss Level [G1] [G6] | 2nd | Polyurethane |
|  |  | 3rd | Polyurethane |
| INT 6.3Y | Polyurethane, Clear, Moisture Cured | 1st | Semi-transparent Stain |
|  | (over stain) | 2nd | Polyurethane |
|  | Gloss Level [G1] [G6] | 3rd | Polyurethane |
|  |  | 4th | Polyurethane |
| INT 6.3Z | Polyurethane, Clear, 2 Component | 1st | Polyurethane |
|  |  | 2nd | Polyurethane |
|  |  | 3rd | Polyurethane |

INT 6.4 – WOOD PANELING

| INT 6.4A | Latex | 1st | Alkyd Primer Sealer |
| --- | --- | --- | --- |
|  | (over alkyd sealer) | 2nd | Latex |
|  | Gloss Level [G1] [G2] [G3] [G4] [G5] [G6] | 3rd | Latex |
| INT 6.4B | Alkyd | 1st | Alkyd Primer Sealer |
|  | (over alkyd sealer) | 2nd | Alkyd |
|  | Gloss Level [G1] [G3] [G5] [G6] | 3rd | Alkyd |
| INT 6.4C | Semi-transparent Stain | 1st | Wood Stain |
|  |  | 2nd | Wood Stain |
| INT 6.4D | Alkyd Varnish | 1st | Semi-transparent Stain |
|  | (over stain) | 2nd | Sanding Sealer / Shellac |
|  | Gloss Level [G1] [G5] [G6] | 3rd | Alkyd Varnish |
|  |  | 4th | Alkyd Varnish |
| INT 6.4E | Polyurethane Varnish | 1st | Semi-transparent Stain |
|  | (over stain) | 2nd | Polyurethane Varnish |
|  | Gloss Level [G4] [G6] | 3rd | Polyurethane Varnish |
|  |  | 4th | Polyurethane Varnish |
| INT 6.4F | Lacquer, Clear | 1st | Semi-transparent Stain |
|  | (over stain) | 2nd | Lacquer Sanding Sealer |
|  | Gloss Level [G1] [G4] [G6] | 3rd | Clear Lacquer |
|  |  | 4th | Clear Lacquer |
| INT 6.4G | Alkyd Varnish | 1st | Sanding Sealer / Shellac |
|  | (over stain) | 2nd | Alkyd Varnish |
|  | Gloss Level [G1] [G5] [G6] | 3rd | Alkyd Varnish |
| INT 6.4H | Lacquer, Pigmented | 1st | Lacquer Sanding Sealer |
|  | Gloss Level [G4] [G5] [G6] | 2nd | Pigmented Lacquer |
|  |  | 3rd | Pigmented Lacquer |
|  |  | 4th | Pigmented Lacquer |
| INT 6.4J | Polyurethane Varnish | 1st | Polyurethane Varnish |
|  | Gloss Level [G4] [G6] | 2nd | Polyurethane Varnish |
|  |  | 3rd | Polyurethane Varnish |
| INT 6.4K | Danish Oil | 1st | Danish Oil |
|  |  | 2nd | Danish Oil |
| INT 6.4L | Multicolor | 1st | Alkyd Primer Sealer |
|  |  | 2nd | Multicolor Base coat |
|  |  | 3rd | Multicolor Pattern Coat |
|  |  | 4th | Clear Coat |
| INT 6.4M | Waterborne Acrylic, Clear | 1st | W. B. Acrylic |
|  | Gloss Level [G4] [G5] [G6] | 2nd | W. B. Acrylic |
|  |  | 3rd | W. B. Acrylic |
| INT 6.4N | Waterborne Light Industrial Coating | 1st | Alkyd Primer Sealer |
|  | Gloss Level [G5] [G6] | 2nd | W. B. Light Industrial Coating |
|  |  | 3rd | W. B. Light Industrial Coating |
| INT 6.4P | Fire Retardant, Pigmented |  | (ULC approved), Apply in strict accordance with Manufacturer’s Instructions |
| INT 6.4Q | Fire Retardant, Clear |  | (ULC approved), Apply in strict accordance with Manufacturer’s Instructions |
| INT 6.4R | Latex | 1st | Latex Primer |
|  | (over latex primer) | 2nd | Latex |
|  | Gloss Level [G5] [G6] | 3rd | Latex |
| INT 6.4S | High Performance Architectural Latex | 1st | Latex Primer Sealer |
|  |  | 2nd | HIPAC Latex |
|  | Gloss level [G2] [G3] [G4] [G5] | 3rd | HIPAC Latex |
| INT 6.4T | Institutional Low Odor / VOC | 1st | Latex Primer |
|  | Gloss level [G1] [G2] [G3] [G4] [G5] [G6] | 2nd | Institutional Low Odor / VOC |
|  |  | 3rd | Institutional Low Odor / VOC |
| INT 6.4U | Waterborne Acrylic, Clear | 1st | Semi-transparent Stain |
|  | (over stain) | 2nd | W. B. Acrylic |
|  | Gloss Level [G4] [G5] [G6] | 3rd | W. B. Acrylic |
|  |  | 4th | W. B. Acrylic |
| INT 6.4V | Polyurethane, Clear, Moisture Cured | 1st | Semi-transparent Stain |
|  | (over stain) | 2nd | Polyurethane |
|  | Gloss Level [G1] [G6] | 3rd | Polyurethane |
|  |  | 4th | Polyurethane |
| INT 6.4W | Lacquer, Clear | 1st | Wood Stain |
|  | (over stain) | 2nd | Lacquer Sanding Sealer |
|  | Gloss Level [G1] [G4] [G6] | 3rd | Clear Lacquer |
|  |  | 4th | Clear Lacquer |
| INT 6.4X | Lacquer, Pigmented | 1st | Lacquer Sanding Sealer |
|  | Gloss Level [G4] [G5] [G6] | 2nd | Pigmented Lacquer |
|  |  | 3rd | Pigmented Lacquer |
|  |  | 4th | Pigmented Lacquer |
| INT 6.4Y | Lacquer, Clear | 1st | Lacquer Sanding Sealer |
|  | Gloss Level [G1] [G4] [G6] | 2nd | Clear Lacquer |
|  |  | 3rd | Clear Lacquer |
|  |  | 4th | Clear Lacquer |

INT 6.5 – WOOD FLOORS AND STAIRS

| INT 6.5A | Alkyd Floor Enamel | 1st | Alkyd Floor Enamel |
| --- | --- | --- | --- |
|  | Gloss level [G4] [G6] | 2nd | Alkyd Floor Enamel |
|  |  | 3rd | Alkyd Floor Enamel |
| INT 6.5B | Polyurethane Varnish | 1st | Wood Stain |
|  | (over stain) | 2nd | Polyurethane Varnish |
|  | Gloss Level G6 | 3rd | Polyurethane Varnish |
|  |  | 4th | Polyurethane Varnish |
| INT 6.5C | Polyurethane Varnish | 1st | Polyurethane Varnish |
|  | Gloss Level G6 | 2nd | Polyurethane Varnish |
|  |  | 3rd | Polyurethane Varnish |
| INT 6.5D | Polyurethane, Moisture Cured | 1st | Polyurethane |
|  | Gloss Level G6 | 2nd | Polyurethane |
|  |  | 3rd | Polyurethane |
| INT 6.5E | Alkyd Game Line Marking | 1st | Alkyd |
|  |  | 2nd | Alkyd |
| INT 6.5F | Epoxy Game Line Marking | 1st | Epoxy |
|  |  | 2nd | Epoxy |
| INT 6.5G | Latex Porch & Floor Enamel | 1st | Alkyd Primer Sealer |
|  | Gloss Level [G3] [G6] | 2nd | Floor Enamel |
|  |  | 3rd | Floor Enamel |
| INT 6.5H | Epoxy Floor Paint, Water Based | 1st | W. B. Epoxy Floor Paint |
|  |  | 2nd | W. B. Epoxy Floor Paint |
|  |  | 3rd | W. B. Epoxy Floor Paint |
| INT 6.5J | Polyurethane, Moisture Cured | 1st | Semi-transparent Stain |
|  | (over stain) | 2nd | Polyurethane |
|  | Gloss Level G6 | 3rd | Polyurethane |
|  |  | 4th | Polyurethane |

INT 6.6 – WOOD SHINGLE AND SHAKE

| INT 6.6A | Latex | 1st | Alkyd Primer Sealer |
| --- | --- | --- | --- |
|  | Gloss Level [G1] [G2] [G3] [G4] [G5] [G6] | 2nd | Latex |
|  |  | 3rd | Latex |
| INT 6.6B | Alkyd | 1st | Alkyd Primer Sealer |
|  | Gloss Level [G1] [G3] [G5] [G6] | 2nd | Alkyd |
|  |  | 3rd | Alkyd |
| INT 6.6C | Semi-transparent Stain | 1st | Semi-transparent Stain |
|  |  | 2nd | Semi-transparent Stain |
| INT 6.6D | Alkyd Solid Colour Stain | 1st | Solid Colour Stain |
|  |  | 2nd | Solid Colour Stain |
| INT 6.6E | Latex Solid Colour Stain | 1st | Latex Wood Primer |
|  |  | 2nd | Latex Stain |
|  |  | 3rd | Latex Stain |
| INT 6.6F | Latex | 1st | Latex Wood Primer |
|  | (over latex primer) | 2nd | Latex |
|  | Gloss Level [G1] [G2] [G3] [G4] [G5] [G6] | 3rd | Latex |
| INT 6.6G | Fire Retardant, Pigmented |  | (ULC approved), Apply in strict accordance with Manufacturer’s Instructions |
| INT 6.6H | Fire Retardant, Clear |  | (ULC approved), Apply in strict accordance with Manufacturer’s Instructions |

INT 6.7 – FIBERGLASS

| INT 6.7A | Latex | 1st | Bonding Primer |
| --- | --- | --- | --- |
|  | Gloss Level [G1] [G2] [G3] [G4] [G5] [G6] | 2nd | Latex |
|  |  | 3rd | Latex |
| INT 6.7B | Alkyd | 1st | Bonding Primer |
|  | Gloss Level [G1] [G3] [G5] [G6] | 2nd | Alkyd |
|  |  | 3rd | Alkyd |
| INT 6.7C | Waterborne Light Industrial Coating | 1st | Bonding Primer |
|  | Gloss Level [G5] [G6] | 2nd | W. B. Light Industrial Coating |
|  |  | 3rd | W. B. Light Industrial Coating |
| INT 6.7D | Epoxy | 1st | Epoxy |
|  |  | 2nd | Epoxy |
|  |  | 3rd | Epoxy |
| INT 6.7E | Polyurethane, Pigmented | 1st | Epoxy |
|  | (over epoxy) | 2nd | Epoxy |
|  |  | 3rd | Polyurethane |
| INT 6.7F | Epoxy, Water Based | 1st | W. B. Epoxy |
|  |  | 2nd | W. B. Epoxy |
|  |  | 3rd | W. B. Epoxy |
| INT 6.7G | Multicolor | 1st | Bonding Primer |
|  |  | 2nd | Multicolor Base coat |
|  |  | 3rd | Multicolor Pattern Coat |
|  |  | 4th | Clear Coat |
| INT 6.7H | High Performance Architectural Latex | 1st | Bonding Primer |
|  |  | 2nd | HIPAC Latex |
|  | Gloss level [G2] [G3] [G4] [G5] | 3rd | HIPAC Latex |
| INT 6.7J | Institutional Low Odor / VOC | 1st | Bonding Primer |
|  | Gloss level [G1] [G2] [G3] [G4] [G5] [G6] | 2nd | Institutional Low Odor / VOC |
|  |  | 3rd | Institutional Low Odor / VOC |

INT 6.8 – PLASTIC

| INT 6.8A | High Performance Architectural Latex | 1st | Bonding Primer |
| --- | --- | --- | --- |
|  |  | 2nd | HIPAC Latex |
|  | Gloss level [G2] [G3] [G4] [G5] | 3rd | HIPAC Latex |
| INT 6.8B | Alkyd | 1st | Bonding Primer |
|  | Gloss Level [G1] [G3] [G5] [G6] | 2nd | Alkyd |
|  |  | 3rd | Alkyd |
| INT 6.8C | Waterborne Light Industrial Coating | 1st | Bonding Primer |
|  | Gloss Level [G5] [G6] | 2nd | W. B. Light Industrial Coating |
|  |  | 3rd | W. B. Light Industrial Coating |
| INT 6.8D | Multicolor | 1st | Bonding Primer |
|  |  | 2nd | Multicolor Base coat |
|  |  | 3rd | Multicolor Pattern Coat |
|  |  | 4th | Clear Coat |
| INT 6.8E | Latex | 1st | Bonding Primer |
|  | Gloss Level [G1] [G2] [G3] [G4] [G5] [G6] | 2nd | Latex |
|  |  | 3rd | Latex |
| INT 6.8F | Institutional Low Odor / VOC | 1st | Bonding Primer |
|  | Gloss level [G1] [G2] [G3] [G4] [G5] [G6] | 2nd | Institutional Low Odor / VOC |
|  |  | 3rd | Institutional Low Odor / VOC |

INT 9.1 – SPRAY TEXTURED CEILIING

| INT 9.1A | Latex, (spray application only) | 1st | Latex |
| --- | --- | --- | --- |
|  | Gloss Level G1 | 2nd | Latex |
| INT 9.1B | Latex, (over alkyd sealer) | 1st | Alkyd Sealer |
|  | Gloss level [G1] [G2] [G3] [G4] [G5] [G6] | 2nd | Latex |
| INT 9.1C | Alkyd | 1st | Alkyd |
|  | Gloss Level G1 | 2nd | Alkyd |
| INT 9.1D | Alkyd | 1st | Alkyd Sealer |
|  | (over alkyd sealer) | 2nd | Alkyd |
|  | Gloss level [G3] [G5] [G6] | 3rd | Alkyd |
| INT 9.1E | Latex, (spray application only) | 1st | Latex |
|  | Gloss level [G1] [G2] [G3] [G4] [G5] | 2nd | Latex |
|  |  | 3rd | Latex |

INT 9.2 – PLASTER AND GYPSUM BOARD

| INT 9.2A | Latex | 1st | Latex Primer Sealer |
| --- | --- | --- | --- |
|  | (over latex sealer) | 2nd | Latex |
|  | Gloss level [G1] [G2] [G3] [G4] [G5] [G6] | 3rd | Latex |
| INT 9.2B | High Performance Architectural Latex | 1st | Latex Primer Sealer |
|  |  | 2nd | HIPAC Latex |
|  | Gloss level [G2] [G3] [G4] [G5] | 3rd | HIPAC Latex |
| INT 9.2C | Alkyd | 1st | Latex Primer Sealer |
|  | (over latex sealer) | 2nd | Alkyd |
|  | Gloss level [G1] [G3] [G5] [G6] | 3rd | Alkyd |
| INT 9.2E | Epoxy | 1st | Latex Primer Sealer |
|  |  | 2nd | Epoxy |
|  |  | 3rd | Epoxy |
| INT 9.2F | Epoxy, Water Based | 1st | Latex Primer Sealer |
|  |  | 2nd | W. B. Epoxy |
|  |  | 3rd | W. B. Epoxy |
| INT 9.2G | Multicolor | 1st | Primer Sealer |
|  |  | 2nd | Multicolor Base coat |
|  |  | 3rd | Multicolor Pattern Coat |
|  |  | 4th | Clear Coat |
| INT 9.2H | Fire Retardant Coating |  | (ULC approved), Apply in strict accordance with Manufacturer’s Instructions |
|  | (clear or pigmented) |  |  |
| INT 9.2J | Latex Fire Retardant Coating |  | (ULC approved), Apply in strict accordance with Manufacturer’s Instructions |
|  | (clear or pigmented) |  |  |
| INT 9.2K (Plaster Only) | Latex | 1st | Alkyd Primer Sealer |
|  | (over alkyd primer) | 2nd | Latex |
|  | Gloss Level [G1] [G2] [G3] [G4] [G5] | 3rd | Latex |
| INT 9.2L | Waterborne Light Industrial Coating | 1st | Latex Primer Sealer |
|  | Gloss Level [G5] [G6] | 2nd | W. B. Light Industrial Coating |
|  |  | 3rd | W. B. Light Industrial Coating |
| INT 9.2M | Institutional Low Odor / VOC | 1st | Latex Primer Sealer |
|  | Gloss level [G1] [G2] [G3] [G4] [G5] [G6] | 2nd | Institutional Low Odor / VOC |
|  |  | 3rd | Institutional Low Odor / VOC |

INT 9.3 – ACOUSTIC PANELS AND TILES

| INT 9.3A | Latex | 1st | Latex |
| --- | --- | --- | --- |
|  | Gloss Level G1 | 2nd | Latex |
| INT 9.3B | Latex | 1st | Alkyd Sealer |
|  | (over alkyd sealer) | 2nd | Latex |
|  | Gloss level [G1] [G2] [G3] [G4] [G5] |  |  |
| INT 9.3C | Alkyd | 1st | Alkyd |
|  | Gloss Level G1 | 2nd | Alkyd |
| INT 9.3D | Institutional Low Odor / VOC | 1st | Institutional Low Odor / VOC |
|  | Gloss level [G1] [G2] [G3] [G4] [G5] [G6] | 2nd | Institutional Low Odor / VOC |
| INT 9.3E | High Performance Architectural Latex | 1st | HIPAC Latex |
|  |  | 2nd | HIPAC Latex |
|  | Gloss level [G2] [G3] [G4] [G5] |  |  |

INT 10.1 – CANVAS AND COTTON COVERINGS

| INT 10.1A | Latex | 1st | Latex Primer Sealer |
| --- | --- | --- | --- |
|  | Gloss level [G1] [G2] [G3] [G4] [G5] [G6] | 2nd | Latex |
|  |  | 3rd | Latex |
| INT 10.1B | Alkyd | 1st | Latex |
|  | Gloss Level [G1] [G3] [G5] [G6] | 2nd | Alkyd |
|  |  | 3rd | Alkyd |
| INT 10.1C | Aluminum Paint | 1st | Latex Primer Sealer |
|  |  | 2nd | Aluminum Paint |
|  |  | 3rd | Aluminum Paint |
| INT 10.1D | Institutional Low Odor / VOC | 1st | Latex Primer Sealer |
|  | Gloss level [G1] [G2] [G3] [G4] [G5] [G6] | 2nd | Institutional Low Odor / VOC |
|  |  | 3rd | Institutional Low Odor / VOC |

INT 10.2 – BITUMINOUS COATED SURFACES

| INT 10.2A | Latex | 1st | Rust Inhibitive Primer |
| --- | --- | --- | --- |
|  | Gloss level [G1] [G2] [G3] [G4] [G5] [G6] | 2nd | Latex |
|  |  | 3rd | Latex |
| INT 10.2B | Alkyd | 1st | Rust Inhibitive Primer |
|  | Gloss Level [G1] [G3] [G5] [G6] | 2nd | Alkyd |
|  |  | 3rd | Alkyd |
| INT 10.2C | Aluminum Paint | 1st | Rust Inhibitive Primer |
|  |  | 2nd | Aluminum Paint |
|  |  | 3rd | Aluminum Paint |

- END OF SECTION